|  | MINISTRY OF EDUCATION AND TRAINING |
| --- | --- |

FPT UNIVERSITY

Capstone Project Document

Designing and making A Lynxmotion A-POD robot controller

Hexapod Team. Group Members; - Phan Anh Dũng Cường - Nguyễn Minh Quân. Supervisor; M.Si. Trần Khánh Ninh. Ext Supervisor. Capstone Project code; APOD.

- Ho Chi Minh City, 09/2013 -

# Table of Contents

Table of Contents	2

List of Figures	8

List of Tables	11

A.	INTRODUCTION	14

I. PROJECT INFORMATION	14

II. TEAM MEMBER	14

III. APOD INTRODUCTION	14

1. HEXAPOD	14

2. LYNXMOTION APOD	16

IV. EXISTING SOLUTIONS	17

V. PROJECT INTRODUCTION	18

VI. APPROACHES	19

VII. OTHER	19

1. WHY WE NEED APOD ROBOT?	19

2. PRODUCT	19

B.	SOFTWARE PROJECT MANAGEMENT PLAN (SPMP)	20

I. PROBLEM DEFINITION	20

1. NAME OF THIS CAPSTONE PROJECT	20

2. PROBLEM ABSTRACT	20

3. PROBLEM OVERVIEW	20

3.1. The Current System	20

3.2. The Propose System	20

3.3. Boundaries of The System	21

3.4. Developing Environment	21

3.4.1 Hardware environment:	21

3.4.2 Software environment:	21

II. PROJECT ORGANIZATION	22

1. SOFTWARE PROCESS MODEL	22

2. ROLES AND RESPONSIBILITIES	23

3. TOOL AND TECHNIQUES	23

3.1. For Development	23

3.1.1 Hardware environment:	23

3.1.2 Software environment:	23

3.2. For Management	23

3.3. For Communication	23

III. PROJECT MANAGEMENT PLAN	24

1. Task	24

1.1. Initiating and Planning:	24

1.2. Software Requirement Analysis	24

1.3. Creating Software Design Description	24

1.4. Coding	25

1.5. Testing	25

1.6. Deployment	25

2. Task sheet	26

3. All meeting minutes	27

IV. CODING CONVENTION	27

C.	SOFTWARE REQUIREMENTS SPECIFICATION (SRS)	28

I. USER REQUIREMENT SPECIFICATIONS	28

II. SYSTEM REQUIREMENT SPECIFICATIONS	28

1. Hardware requirement	28

2. Software interface	28

3. External interface	28

4. Functional requirement	28

5. Non-functional requirement	29

III. USE-CASE DIAGRAM AND USE-CASE SPECIFICATIONS	29

1. Use-case Diagram	29

2. Use-case Specifications	30

2.1. Sequences Control	30

2.1.1 Add State	30

2.1.2 Remove states	31

2.1.3 Import States	32

2.1.4 Export States	34

2.2. Direct Control	35

2.2.1 Mode Selection	35

a)	Normal Input	35

b)	Gamepad Input	36

2.2.2 Movement Control	38

a)	Move Forward	38

b)	Move Backward	39

c)	Turn Left	41

d)	Turn Right	42

e)	Body Lift	44

f)	Body Drop	45

g)	Squeeze Left	46

h)	Squeeze Right	48

i)	Manual Grip	49

2.3. Object tracking	51

2.3.1 Auto Grip	51

2.3.2 Choose Target	52

2.3.3 Change Target	54

2.3.4 Lock Target	55

IV. SOFTWARE SYSTEM ATTRIBUTES	56

1. Reliability	56

2. Availability	57

3. Security	57

4. Maintainability	57

5. Portability	57

6. Performance	57

D.	SOFTWARE DESIGN DESCRIPTION (SDD)	57

I. DESIGN OVERVIEW	57

1. Introduction	57

2. Architecture	58

3. User interface	58

II. COMPONENTS	59

1. Central processing:	59

1.1. Bluetooth module (HC 05)	59

1.2. STM32F4 Discovery	60

2. Servos control & Sensor reader	61

2.1. HG-645MG Servo	61

2.2. SSC32 with ATMEGA328P	62

2.3. Ultrasonic ranging module (HC SR04)	62

III. IMPLEMENTATION APPROACHES	63

1. Algorithms and Mechanism	63

1.1. RC Servo Control	63

1.1.1 HG-645MG Specification	63

1.1.2 How does it work?	64

1.1.3 Servo’s Speed Control	65

1.2. Sensor Reader (HC-SR04)	66

1.2.1 HC-SR04 Specifications	66

1.2.2 How to read distance value?	67

1.3. Multi-legs Control	68

1.3.1 Tripod Gait	68

1.3.2 Wave Gait	68

1.3.3 Ripple Gait	69

1.3.4 Implementation Approaches	70

a)	Walking In Straight Line	70

b)	Turning Left – Right	75

2. SSC32 Board	77

2.1. Schematic	77

2.2. Technical Point	78

2.2.1 Atmega328P Microcontroller (MCU)	78

a)	Overview	78

b)	Pin Configuration	80

c)	Block Diagram	81

2.2.2 General Purposes Input / Output (GPIO)	81

a)	Configurations	82

b)	Register Description	82

2.2.3 Interrupts	84

2.2.4 Timer	84

a)	Usage and Purposes	84

b)	Register Description	85

2.2.5 Universal Synchronous and Asynchronous serial Receiver and Transmitter (USART)	87

a)	Usage and Purposes	87

b)	Baud Rate	87

c)	Register Description	88

2.2.6 Serial Peripheral Interface (SPI)	91

a)	Usage and Purposes	91

b)	Register Description	91

2.2.7 Shift Register (74HC595)	92

a)	Overview	92

b)	How does it work?	92

2.2.8 SPI - Shift Register interface	93

a)	Overview	93

b)	How does it work?	93

3. STM32F4 Discovery Board	95

IV. ACTIVITY AND CLASS STRUCTURE	95

1. Activity	95

1.1. APOD Controller (HMI Application)	95

1.2. STM32F4 Discovery (Central processing board)	97

1.3. SSC32 Board (Servos control and Sensor reader)	98

2. Structure	99

2.1. APOD Controller (HMI Application)	99

2.2. Central Processing Module (STM32F4)	99

2.3. Servo Control & Sensor Reader Module (SSC32)	99

V. SEQUENCE DIAGRAM	99

1. Embedded Controller	99

1.1. Forward	99

1.2. Backward	100

1.3. Turn Left	101

1.4. Turn Right	102

2. Use-cases Sequences	103

2.1. Sequences Control	103

2.1.1 Add State	103

2.1.2 Remove States	104

2.1.3 Import States	104

2.1.4 Export States	105

2.2. Direct Control	106

2.2.1 Mode Selection	106

a)	Normal Input	106

b)	Gamepad Input	107

2.2.2 Movement Control	108

a)	Forward	108

b)	Backward	108

c)	Turn Left	109

d)	Turn Right	109

e)	Body Lift	110

f)	Body Drop	110

g)	Squeeze Left	111

h)	Squeeze Right	111

2.3. Object Tracking	112

2.3.1 Auto Grip	112

2.3.2 Choose target	113

2.3.3 Change Target	114

2.3.4 Lock Target	115

VI. USER INTERFACE DESIGN	116

1. Live Control	116

2. Sequence Player	117

3. Configuration	118

4. Object Extractor	119

E.	SOFTWARE TEST DOCUMENTATION (STD)	120

I. Introduction	120

1. Overview	120

2. Test Approach	120

II. Test Plan	120

1. Features to be tested	120

2. Features not to be tested	120

3. Test Environment	121

III. Test Cases	121

1. Functional Test	121

2. UI Test	15

F.	SOFTWARE USER MANUAL (SUM)	1

G.	APPENDIX	1

I. Glossary	1

II. References	1

# List of Figures

Figure A-1 CH3-R Walking Robot	15

Figure A-2 AH2 Walking Robot	15

Figure A-3 Lynxmotion APOD	16

Figure A-4 Flowbotics Studio example 1	17

Figure A-5 Flowbotics Studio example 2	17

Figure A-6 Lynxmotion Visual Sequences	18

Figure B-1 System Boundary Overview	21

Figure B-2 Spriral model	22

Figure B-3 Project plan	27

Figure B-4 Project plan	27

Figure C-1: Use cases diagram	29

Figure C-2 Add State Use-case diagram	30

Figure C-3 Remove States Use-case diagram	31

Figure C-4 Import States Use-case diagram	32

Figure C-5 Export States Use-case diagram	34

Figure C-6 Normal input Use-case diagram	35

Figure C-7 Gamepad Input Use-case diagram	36

Figure C-8 Move Forward Use-case diagram	38

Figure C-9 Move Backward Use-case diagram	39

Figure C-10 Turn Left Use-case diagram	41

Figure C-11 Turn Right Use-case diagram	42

Figure C-12 Body Lift Use-case diagram	44

Figure C-13 Body Drop Use-case diagram	45

Figure C-14 Squeeze Left Use-case diagram	46

Figure C-15 Squeeze Right Use-case diagram	48

Figure C-16 Manual Grip Use-case diagram	49

Figure C-17 Auto Grip Use-case diagram	51

Figure C-18 Choose Target Use-case diagram	52

Figure C-19 Change Target Use-case diagram	54

Figure C-20 Lock Target Use-case diagram	55

Figure D-1 APOD Controller System Architecture	58

Figure D-2 Live Control User interface design	58

Figure D-3 Sequence Player user interface design	59

Figure D-4 Bluetooth Communication module HC05	60

Figure D-5 STM32F4 Discovery Board	61

Figure D-6 HC 645MG Servo	62

Figure D-7 SSC32 Board	62

Figure D-8 HCSR04 Distance Sensor	63

Figure D-9 RC Servo block	64

Figure D-10 Servo Signal illustration	64

Figure D-11 HC 645MG Signal Details	65

Figure D-12 HC-SR04 Specifications	66

Figure D-13 HC-SR04 Best practice angle	66

Figure D-14 HC-SR04 Operating sequence	67

Figure D-15 Tripod Gait illustration	68

Figure D-16 Wave Gait illustration	69

Figure D-17 Wave Gait step order	69

Figure D-18 Ripple Gait illustration	70

Figure D-19 APOD leg position block	71

Figure D-20 APOD leg swing diagram	72

Figure D-21 APOD Leg full step diagram	72

Figure D-22 Tripod A at Front position	73

Figure D-23 Tripod A at Neutral position	74

Figure D-24 Tripod A at Rear position	74

Figure D-25 Tripod A at Left position	76

Figure D-26 Tripod A at Right position	76

Figure D-27 SSC32 Schematic	78

Figure D-28 ATmega328p Microcontroller	79

Figure D-29 ATmega328p 28 Pins DIP	80

Figure D-30 ATmega328p Block Diagram	81

Figure D-31 MCU Control Register	82

Figure D-32 PORT B Register Description	83

Figure D-33 PORT C Register Description	83

Figure D-34 PORT D Register Description	84

Figure D-35 Timer 1 Control register A	85

Figure D-36 Timer 1 Control register B	85

Figure D-37 Timer 1 Control register C	85

Figure D-38 Timer 1 Counter	86

Figure D-39 Output Compare Register A	86

Figure D-40 Output Compare Register B	86

Figure D-41 Input Capture Register	86

Figure D-42 Timer/Counter1 Interrupt Mask Register	86

Figure D-43 Timer 1 Interrupt Flog register	87

Figure D-44 Baud Rate setup	87

Figure D-45 USART I/O Data register	88

Figure D-46USART Control and Status Register A	88

Figure D-47 USART Control and Status register B	89

Figure D-48 USART Control and Status register C	89

Figure D-49 USART Baud Rate register	90

Figure D-50 Baud Rate Calculation	90

Figure D-51 SPI Control register	91

Figure D-52 SPI Status register	91

Figure D-53 SPI Data register	91

Figure D-54 74HC595 Block diagram	92

Figure D-55 SPI Diagram	93

Figure D-56 SPI- Shift Register	94

Figure D-57	96

Figure D-58	97

Figure D-59	98

Figure D-61	99

Figure D-62	100

Figure D-63	101

Figure D-64	102

Figure D-65	103

Figure D-66	104

Figure D-67	104

Figure D-68	105

Figure D-69	106

Figure D-70	107

Figure D-71	108

Figure D-72	108

Figure D-73	109

Figure D-74	109

Figure D-75	110

Figure D-76	110

Figure D-77	111

Figure D-78	111

Figure D-79	112

Figure D-80	113

Figure D-81	114

Figure D-82	115

Figure D-83 Live Control Interface Design	116

Figure D-84 Sequence Player Interface Design	117

Figure D-85 Configuration Interface Design	118

Figure D-86 Object Extractor Interface Design	119

# List of Tables

Table A-1 Team Members	14

Table B-1  Roles and Responsibilities	23

Table B-2 Initiating and Planning	24

Table B-3 Software Requirement Analysis	24

Table B-4 Creating Software Design Description	25

Table B-5 Coding	25

Table B-6 Testing	25

Table B-7 Deployment	26

Table C-1	31

Table C-2	32

Table C-3	34

Table C-4	35

Table C-5	36

Table C-6	38

Table C-7	39

Table C-8	40

Table C-9	42

Table C-10	43

Table C-11	45

Table C-12	46

Table C-13	48

Table C-14	49

Table C-15	51

Table C-16	52

Table C-17	53

Table C-18	55

Table C-19	56

Table D-1	71

Table D-2 APOD Leg full forward step	73

Table D-3 Forward Sequence	75

Table D-4 Backward Sequence	75

Table D-5 Turn Left Sequence	77

Table D-6 Turn Right Sequence	77

Table D-7 Interrupts Usage	84

Table D-8 Baud Rate setup details	88

Table D-9 Live Control Interface Design Description	116

Table D-10 Sequence Player Interface Design Description	117

Table D-11 Configuration Interface Design Description	118

Table D-12 Object Extractor Interface Design Description	119

This page intentionally left blank.

# INTRODUCTION

## PROJECT INFORMATION

- Project name: Lynxmotion A-Pod  robot controller

- Project code: APOD

- Product type: Embedded robot controller

- Timeline: from September 2013 to December 2013

## TEAM MEMBER

| No. | Name | Role | Contact |
| --- | --- | --- | --- |
| 1 | Phan Anh Dũng Cường | Leader | - Phone: 0972708318 - Email: cuongpadse60774@fpt.edu.vn |
| 2 | Nguyễn Minh Quân | Member | - Phone: 0915699635 - Email: quannm60344@fpt.edu.vn |

Table A-1 Team Members

## APOD INTRODUCTION

### HEXAPOD

The trademarked name “hexapod” (by Geodetic Technology) was originally for Stewart platforms used in machine tools. However, the term is now used for 6-jack platforms outside of the machine tool area, since it simply means "six legs".

In the scope of this document, “hexapod” will represent a structure with six legs which can be used for variety of Robotics application.

Variety of robotics hexapod:

Figure A-1 CH3-R Walking Robot

Figure A-2 AH2 Walking Robot

### LYNXMOTION APOD

Figure A-3 Lynxmotion APOD

- Insect inspired body.

- Assembly with 25 servos (motors) and PVC materials to form 6 legs, mandible grippers (with force sensor feature) and a tail.

- The three DOF (degree of freedom) leg design help the APOD moving in a variety of directions.

- Servos are directly controlled by SSC-32 board which receive command via RS232 interface from Bot board II / PC or any other devices communicate using RS232.

## EXISTING SOLUTIONS

- Flowbotics Studio: $39.9

Figure A-4 Flowbotics Studio example 1

Figure A-5 Flowbotics Studio example 2

- Lynxmotion Visual Sequences: $39.94

Figure A-6 Lynxmotion Visual Sequences

## PROJECT INTRODUCTION

This project focus on programming on microprocessor unit (MCU) to developing, control the A-Pod robot. The BASIC is let him move forward, backward, right, left. Thus, we concern on control over Bluetooth.  Through Camera put on robot control it/ or automatic function, connect with Sensor to discover obstacles.

Abstracts purpose: Design the controller board motor servo using microcontroller to control Lynxmotion A-Pod  robot  with available mechanical. Implement the hardware and software on PC to control Lynxmotion. The system requires Microcontroller ARM LPC,AT91SAM,PIC,AVR. Finding critical points and find best way algorithms.

## APPROACHES

- Building SSC32 firmware with ATMega328p microcontroller to control 25 servos.

- Using Bot Board II  as universal sensor reader to serve automation purposes (additional sensors will be needed).

- Additional STM32F4 Discovery using ARM will be used as a central processing module which communicate with other device via Bluetooth.

- IP Camera to provide an insight view for user.

- A friendly HMI (Human Machine Interface) program will be provided for user to control the APOD with ease.

## OTHER

### WHY WE NEED APOD ROBOT?

The main purpose of A-Pod Robot is to be used at home with variable simple tasks. The Project ‘s scope is to control A-Pod from a distance via a HCI program on PC (or mobile devices). A-Pod can perform task without the present of human and report back working  information when needed.

### PRODUCT

- HMI Program on PC (Possible extend: Android/IOS).

- Servo Control Module Software (SSC32 firmware).

- Sensor Service Module  Software (Bot Board II firmware)

- Central Processing Module Software (STM32F4 Discovery).

# SOFTWARE PROJECT MANAGEMENT PLAN (SPMP)

## PROBLEM DEFINITION

### NAME OF THIS CAPSTONE PROJECT

  - Project name: Lynxmotion A-Pod  robot controller

  - Project code: APOD

  - Product type: Embedded robot controller

### PROBLEM ABSTRACT

As mentioned above, this project will focus on implement a new control system for APOD using Bluetooth/wireless. The main purpose of new controller system is to make the APOD more flexible. In order to achieve that purpose, the final product should have those characteristics:

  - Friendly HMI interface.

  - Easy to use, easy to learn.

  - Different user experience: comfort and high interaction.

  - Adaptable to new mechanical constraint, prepared for complete automation.

### PROBLEM OVERVIEW

#### The Current System

Current APOD control system can be control using either USB-to-serial cable or an PS2 interface. The SSC32 will be responsible for direct controlling of motor servos.

- Advantages:

  - Direct command.

  - No delay due to no-subsystem needed (USB-to-serial control).

  - Simple architecture

- Limitation:

  - Very limited range of control due to “wire” problem

  - A considerable amount of wire/cable can be obstacles for APOD movement.

  - Domain or mechanical knowledge is required to perform.

  - On-sight performance: APOD must stay insight of user viewpoint.

#### The Propose System

The system consists of 4 module:

  - Servos Control Module (SSC-32): Directly control servo ‘s movement.

  - Sensor Service Module (Bot Board II): receive all kind of sensor signal and value.

  - Central Processing Module (STM32F4 Discovery) plus Bluetooth module

  - HMI software on PC

Main features will be provided:

  - Basic movement: forward, backward, turn left, turn right.

  - High interactive user interface.

#### Boundaries of The System

Figure B-1 System Boundary Overview

  - The APOD can perform only within Bluetooth or Wireless device ‘s range

  - Any interfere while receiving command (transmitting via Bluetooth) will cause the APOD to misbehave or unpredictable movement.

#### Developing Environment

##### Hardware environment:

- Lyxnmotion APOD

- IP camera, Bluetooth device, Distance sensor.

- 4 laptops with appropriate configuration for embedded development.

##### Software environment:

- Developing environment ARV studio 4 for main board programming.

- Developing environment Visual studio 2012 with C# language for user interface programming.

## PROJECT ORGANIZATION

### SOFTWARE PROCESS MODEL

The process model used for developing this project is Spiral Model.

The spiral model is a software development process combining elements of both design and prototyping-in-stages, in an effort to combine advantages of top-down and bottom-up concepts. Also known as the spiral lifecycle model (or spiral development), it is a systems development method (SDM) used in information technology (IT). This model of development combines the features of the prototyping and the waterfall model. The spiral model is intended for large, expensive and complicated projects.

Figure B-2 Spriral model

### ROLES AND RESPONSIBILITIES

| No. | Full name | Team Role | Responsibilities |
| --- | --- | --- | --- |
| 1 | Trần Khánh Ninh | Supervisor | Define business Support in technical issues |
| 1 | Phan Anh Dũng Cường | Team Leader | Prioritize work Answer question and deliver direction Facilitate productivity – maximize team performance Complete all individual work |
| 3. | Nguyễn Minh Quân | Team Member | Commit individual product on time Support each other to complete team work |

Table B-1  Roles and Responsibilities

### TOOL AND TECHNIQUES

#### For Development

##### Hardware environment:

- Lynxmotion APOD

- IP camera, Bluetooth device, Distance sensor.

- 4 laptops with appropriate configuration for embedded development.

##### Software environment:

- Developing environment ARV studio 4 for main board programming.

- Developing environment Visual studio 2010 with C# language for user interface programming.

- Developing environment Keilc studio for program ARM.

#### For Management

- Microsoft Project: Task tracking

- Tortoise SVN: Source version control

- Google Code: Connect, synchronize source code and documents

- Beyond Compare: Manage changes in source files

#### For Communication

- Gmail report

- Skype chat system

- CMS system

## PROJECT MANAGEMENT PLAN

### Task

#### Initiating and Planning:

| Description | Registering project, kick-off meeting and planning |
| --- | --- |
| Output | Registered project, team spirit, overview plan |
| Deliverables | Draft project plan |
| Effort (man-month) | 0.8 mm |
| Dependencies and Constrains | N/A |
| Risks | Some members may be absent |

Table B-2 Initiating and Planning

#### Software Requirement Analysis

| Description | Analyzing software requirements based on available hexapod’s hardware to create software requirements specification document |
| --- | --- |
| Output | Software Requirement Specification document |
| Deliverables | SRS document file |
| Effort (man-month) | 1.5 mm |
| Dependencies and Constrains | N/A |
| Risks | - Lack of knowledge about hexapod’s hardware |

Table B-3 Software Requirement Analysis

#### Creating Software Design Description

| Description | Designing the controller for hexapod robot based on actual requirements |
| --- | --- |
| Output | Architecture design, circuits diagram, board diagram, algorithms and design specification |
| Deliverables | SDD document |
| Effort (man-month) | 3.0 mm |
| Dependencies and Constrains | Completion of SRS |
| Risks | - Choosing inappropriate algorithms and design patterns - The hardware is hard to maintain - Causing high coding efforts |

Table B-4 Creating Software Design Description

#### Coding

| Description | Designing the controller for hexapod robot based on actual requirements |
| --- | --- |
| Output | Architecture design, circuits diagram, board diagram, algorithms and design specification |
| Deliverables | SDD document |
| Effort (man-month) | 3.0 mm |
| Dependencies and Constrains | Completion of SRS |
| Risks | - Choosing inappropriate algorithms and design patterns - The hardware is hard to maintain - Causing high coding efforts |

Table B-5 Coding

#### Testing

| Description | Creating test case and execute test |
| --- | --- |
| Output | Test plan, test case document, test report, all tested modules and tested system |
| Deliverables | Test plan, test report |
| Effort (man-month) | 2.2 mm |
| Dependencies and Constrains | Completion of SRS, SDD, coding |
| Risks | - Lack of professional testers in team - Unit test may not be performed thoroughly causing spending many efforts in system test phase. - Hardware ‘s limitations and errors - ES testing is different with IS testing |

Table B-6 Testing

#### Deployment

| Description | Deploying system include : hexapod robot and hexapod controller simulator |
| --- | --- |
| Output | Software packages, user manual |
| Deliverables | Software packages, user manual |
| Effort (man-month) | 0.3 mm |
| Dependencies and Constrains | Completion of all other tasks |
| Risks | - Hardware’s errors |

Table B-7 Deployment

### Task sheet

Figure B-3 Project plan

Figure B-4 Project plan

### All meeting minutes

<to be updated>

## CODING CONVENTION

The following rules follow:

- The standard rules for developing application using C/C++ (http://users.ece.cmu.edu/~eno/coding/CCodingStandard.html)

# SOFTWARE REQUIREMENTS SPECIFICATION (SRS)

## USER REQUIREMENT SPECIFICATIONS

- A-Pod should be able to walk freely in any direction.

- Controlling via a Serial connection like Bluetooth, or directly through a PS2 controller.

- A-Pod should be able to grab small things like a Coke.

- A-Pod should be able to recording video.

- A-Pod should be able to detect objects stand (within 30cm) in the ways and response back.

- Controller should be simple and easy.

- After receive movement signal, A-Pod should move within 2 seconds

## SYSTEM REQUIREMENT SPECIFICATIONS

### Hardware requirement

- 25 HS-645MG servo

- 1 SSC-32 servo controller

- 1 Bot Board II with Basic Atom Pro 28 microcontroller

- 1 HC-SR04 supersonic sensor

- 1 STM32F4 ARM

- Laptops with Bluetooth Device

- 1 PS2 controller

- Few serial cables, adapters , jumpers…

### Software interface

- Menu-driven design with : button, dialog box, screen.

- Button : movement button on the right, setting like PS2 controller.

- Dialog box : on the left, display A-Pod ‘s currently speed, object stand in way and distance.

- Screen : center of menu controller, display view from camera.

### External interface

- PS2 Controller

### Functional requirement

- Direction control : user can direct the A-Pod to turn left, right, go forward or backward.

- Camera view control : user can see the view sending by the A-Pod camera.

- Grab control : user can direct the A-Pod to grab things.

- Remote control : enable remote controlling, help the user control the A-Pod through a programmed Bluetooth device.

### Non-functional requirement

- The A-Pod should response and move after receiving order within 2 seconds.

- The longest range for leg movement should be 30 centimeter, 35 degree.

- Detected range should be larger than 2 meter.

- View from camera should be refresh  every 0.5 second.

- The A-Pod must be compatible with the following batteries and chargers:

  - NiCad & Ni-MH Universal Smart Charger (USC-02)

  - Volt Ni-MH 2800mAh Battery Pack

- All cable, jumper, adapter should be corrected connect

## USE-CASE DIAGRAM AND USE-CASE SPECIFICATIONS

### Use-case Diagram

Figure C-1: Use cases diagram

### Use-case Specifications

#### Sequences Control

##### Add State

Figure C-2 Add State Use-case diagram

| ADD STATE USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC001 | Use-case Version |  | 1.0 |
| Use-case Name | Add State |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Add a state (move) to sequence Goal: New state is added to sequence Triggers User click [Add] button Preconditions: - Sequence Player feature is selected - Interval (if required) stay within valid range Post Conditions: NONE Main Success Scenario: - User select move to add to sequence - Enter interval value (if available) - Click [Add] Alternative Scenario: NONE Exceptions: NONE Relationships: NONE |  |  |  |  |

Table C-1

##### Remove states

Figure C-3 Remove States Use-case diagram

| REMOVE STATES USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC002 | Use-case Version |  | 1.0 |
| Use-case Name | Remove States |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Remove selected states from current sequence Goal: All selected state to be removed Triggers User click [Remove] Button Preconditions: - Sequence Player feature is selected Post Conditions: NONE Main Success Scenario: - User select all move to be removed from sequence - Click [Remove] Alternative Scenario: NONE Exceptions: NONE Relationships: NONE |  |  |  |  |

Table C-2

##### Import States

Figure C-4 Import States Use-case diagram

| IMPORT STATES USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC003 | Use-case Version |  | 1.0 |
| Use-case Name | Import States |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Import existing sequence from file Goal: Current sequence is replace by new one from file Triggers User click [Import] Button Preconditions: - Sequence Player feature is selected Post Conditions: NONE Main Success Scenario: - Click [Import] button - Browse to location contain sequence file (*.xml) - Select file & click [Ok] - Validate new sequence content - Replace current sequence if valid Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-3

##### Export States

Figure C-5 Export States Use-case diagram

| EXPORT STATES USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC004 | Use-case Version |  | 1.0 |
| Use-case Name | Export States |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Export current sequence to file Goal: New Sequence file Triggers User click [Export] button Preconditions: - Sequence Player feature is selected Post Conditions: NONE Main Success Scenario: - Click [Export] button - Browse to location contain sequence file (*.xml) - Click [Ok] Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-4

#### Direct Control

##### Mode Selection

###### Normal Input

Figure C-6 Normal input Use-case diagram

| NORMAL INPUT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC005 | Use-case Version |  | 1.0 |
| Use-case Name | Normal Input Selection |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Using normal input method (keyboard or Mouse) Goal: Normal input is selected Triggers User click [Normal Mode] button Preconditions: - Live control feature is selected Post Conditions: NONE Main Success Scenario: - User click [Normal Mode] button - Configuration is checked - Establish Bluetooth connection Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-5

###### Gamepad Input

Figure C-7 Gamepad Input Use-case diagram

| GAMEPAD INPUT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC006 | Use-case Version |  | 1.0 |
| Use-case Name | Gamepad Input Selection |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Using Gamepad input method (Gamepad) Goal: Gamepad input is selected Triggers User click [Gamepad Mode] button Preconditions: - Live control feature is selected - Gamepad is connected Post Conditions: NONE Main Success Scenario: - User click [Gamepad Mode] button - Configuration is checked - Gamepad device connection is checked - Establish Bluetooth connection Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-6

##### Movement Control

###### Move Forward

Figure C-8 Move Forward Use-case diagram

| MOVE FORWARD USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC007 | Use-case Version |  | 1.0 |
| Use-case Name | Move Forward |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to move forward Goal: APOD walking in forward direction Triggers User click [Forward] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Forward] button () Alternative Scenario: - Alt 1: press “W” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-7

###### Move Backward

Figure C-9 Move Backward Use-case diagram

| MOVE BACKWARD USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC008 | Use-case Version |  | 1.0 |
| Use-case Name | Move Backward |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to move Backward Goal: APOD walking in Backward direction Triggers User click [Backward] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Backward] button () Alternative Scenario: - Alt 1: press “S” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-8

###### Turn Left

Figure C-10 Turn Left Use-case diagram

| TURN LEFT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC009 | Use-case Version |  | 1.0 |
| Use-case Name | Turn Left |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to turn to the left Goal: APOD turn in Left direction Triggers User click [Turn Left] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Turn Left] button () Alternative Scenario: - Alt 1: press “A” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-9

###### Turn Right

Figure C-11 Turn Right Use-case diagram

| TURN RIGHT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC010 | Use-case Version |  | 1.0 |
| Use-case Name | Turn Right |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to turn to the Right Goal: APOD turn in Right direction Triggers User click [Turn Right] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Turn Right] button () Alternative Scenario: - Alt 1: press “D” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-10

###### Body Lift

Figure C-12 Body Lift Use-case diagram

| BODY LIFT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC011 | Use-case Version |  | 1.0 |
| Use-case Name | Body Lift |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to Lift up Goal: APOD Lift body up Triggers User click [R1] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected Post Conditions: NONE Main Success Scenario: - User click on [R1] button () Alternative Scenario: - Alt 1: press “U” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-11

###### Body Drop

Figure C-13 Body Drop Use-case diagram

| BODY DROP USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC012 | Use-case Version |  | 1.0 |
| Use-case Name | Body Drop |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to Drop down Goal: APOD Drop body Down Triggers User click [R2] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected Post Conditions: NONE Main Success Scenario: - User click on [R1] button () Alternative Scenario: - Alt 1: press “O” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-12

###### Squeeze Left

Figure C-14 Squeeze Left Use-case diagram

| SQUEEZE LEFT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC013 | Use-case Version |  | 1.0 |
| Use-case Name | Squeeze Left |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to squeeze to the left Goal: APOD Drop Squeeze left Triggers User click [Turn Left] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L2] button is pressed Post Conditions: NONE Main Success Scenario: - User press [L2] button () - User click on [Turn Left] button () Alternative Scenario: - Alt 1: press “A” key on keyboard while pressing “E” key (if Normal input is selected) - Alt 2: press () key on gamepad while pressing (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-13

###### Squeeze Right

Figure C-15 Squeeze Right Use-case diagram

| SQUEEZE RIGHT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC014 | Use-case Version |  | 1.0 |
| Use-case Name | Squeeze Right |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to squeeze to the right Goal: APOD Drop Squeeze right Triggers User click [Turn Right] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L2] button is pressed Post Conditions: NONE Main Success Scenario: - User press [L2] button () - User click on [Turn Left] button () Alternative Scenario: - Alt 1: press “D” key on keyboard while pressing “E” key (if Normal input is selected) - Alt 2: press () key on gamepad while pressing (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-14

###### Manual Grip

Figure C-16 Manual Grip Use-case diagram

| MANUAL GRIP USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC015 | Use-case Version |  | 1.0 |
| Use-case Name | Manual Grip |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to Grip Mandibles Goal: APOD Mandibles Grip Triggers User click [Grip] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Grip] button () Alternative Scenario: - Alt 1: press “I” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-15

#### Object tracking

##### Auto Grip

Figure C-17 Auto Grip Use-case diagram

| AUTO GRIP USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC016 | Use-case Version |  | 1.0 |
| Use-case Name | Auto Grip |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Auto detect object in range and move forward to grab it. Goal: Object is grabbed Triggers User click [AUTO GRIP] button Preconditions: - Live control feature is selected - Bluetooth connection is established Post Conditions: NONE Main Success Scenario: - User click [AUTO GRIP] button - Auto grip command is sent to APOD Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-16

##### Choose Target

Figure C-18 Choose Target Use-case diagram

| CHOOSE TARGET USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC017 | Use-case Version |  | 1.0 |
| Use-case Name | Choose Target |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Enter tracking mode, choosing target to track Goal: New tracking template is selected Triggers User click [Object Tracking] button Preconditions: - Configuration is initiated - Bluetooth connection is established - IP camera stream is working Post Conditions: NONE Main Success Scenario: - [Object Tracking] button is clicked - Object extractor is showed - User select tracking template from capture frame. - User click [OK] Alternative Scenario: NONE Exceptions: - IO Exception - Time Out Exception Relationships: NONE |  |  |  |  |

Table C-17

##### Change Target

Figure C-19 Change Target Use-case diagram

| ADD POINT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC018 | Use-case Version |  | 1.0 |
| Use-case Name | Change Target |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Change current tracking template Goal: New tracking template is selected Triggers User click [Change target] button Preconditions: - Configuration is initiated - Bluetooth connection is established - IP camera stream is working - Tracking mode is selected Post Conditions: NONE Main Success Scenario: - [Change Target] button is clicked - Object extractor is showed - User select tracking template from capture frame. - User click [OK] Alternative Scenario: NONE Exceptions: - IO Exception - Time Out Exception Relationships: NONE |  |  |  |  |

Table C-18

##### Lock Target

Figure C-20 Lock Target Use-case diagram

| ADD POINT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC019 | Use-case Version |  | 1.0 |
| Use-case Name | Lock Target |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Start tracking target Goal: Reach target (within 60 cm) Triggers User click [Lock] button Preconditions: - Configuration is initiated - Bluetooth connection is established - IP camera stream is working - Tracking template was set Post Conditions: NONE Main Success Scenario: - User click on [Lock] button - APOD keep moving forward (with adjustment) until target is reached - User decide to Grab object or not. Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-19

## SOFTWARE SYSTEM ATTRIBUTES

### Reliability

- Easy to upgrade firmware.

- The hexapod controller can be replaced easily by loaded into chip if the controller has problem.

- The APOD system is guarantee by quality testing in:

  - Stability constraints.

  - Functionality.

  - Reliability.

- It’s mostly depending on hardware reliability.

- Small error margin when moving.

### Availability

- In case of electrical incident, the hexapod system will be shut down and reset automatically.

- Controller has the flexibility that allows changes in hardware design.

- Hexapod Controller can actuate the 6-legs forces to properly position the mobile plate given a desired trajectory.

### Security

- N/A

### Maintainability

- N/A

### Portability

- The hexapod controller is depend on hardware so that hexapod system do not have the portability attribute. However, the hexapod controller can easily be loaded into chips and use in different motors.

- The development environment and the language constructs used ensure portability as much as possible to avoid the limitation of software’s changes each time the hardware is upgraded or replaced.

### Performance

- Hexapod controller can control multi-motors in the same time.

- The current hardware can move correctly

# SOFTWARE DESIGN DESCRIPTION (SDD)

## DESIGN OVERVIEW

### Introduction

This section is created to introduce and give a brief overview of the system design.

The followings information are given in this section:

- System Architecture Design : the system overall structure

- Components Description : detailed info of hardware components

- Class Diagram : entity & attributes

- Sequence Diagram : describe the flow of events

- User Interface Design

### Architecture

Figure D-1 APOD Controller System Architecture

### User interface

Figure D-2 Live Control User interface design

Figure D-3 Sequence Player user interface design

## COMPONENTS

### Central processing:

#### Bluetooth module (HC 05)

Communication module (using Bluetooth) to transmit and receive commands from Controller devices (PC)

Figure D-4 Bluetooth Communication module HC05

#### STM32F4 Discovery

The Center Processing module to control all APOD movements or reaction to a specific event (command or obstacle).

Figure D-5 STM32F4 Discovery Board

### Servos control & Sensor reader

#### HG-645MG Servo

- Metal gear servo that provided the base of APOD movement.

Figure D-6 HC 645MG Servo

#### SSC32 with ATMEGA328P

- A servo controller : contains 32 pin channels of 1uS resolution servo control

- For further information, followings this link : Lynxmotion SSC32

Figure D-7 SSC32 Board

#### Ultrasonic ranging module (HC SR04)

- Ultrasonic module using for distance measurement, discovering obstacles.

Figure D-8 HCSR04 Distance Sensor

## IMPLEMENTATION APPROACHES

### Algorithms and Mechanism

#### RC Servo Control

##### HG-645MG Specification

| Motor Type: | 3 Pole |
| --- | --- |
| Bearing Type: | Dual Ball Bearing |
| Speed (4.8V/6.0V): | 0.24 / 0.20 |
| Torque oz./in. (4.8V/6.0V): | 107 / 133 |
| Torque kg./cm. (4.8V/6.0V): | 8.0 / 10.0 |
| Size in Inches: | 1.59 x 0.77 x 1.48 |
| Size in Millimeters: | 40.39 x 19.56 x 37.59 |
| Weight ounces: | 1.94 |
| Weight grams: | 55 |

##### How does it work?

Unlike the AC/DC motor, Servo motor do not rotate continuously.  Servo motor rotation range usually fall within 90o to 180o (some special servo can rotate up to 360o) and keep holding the torque at the position as long as its power supply remain or until it receive signal to move the torque.

Most of Servos have 3 wires connector. One wire for power supply (4 – 6v). The second wire is for voltage ground. And the third wire is for receiving control signal.

Figure D-9 RC Servo block

The whole idea to control servo is sending the appropriate signal to servo so that it will move to the intended position. The signal to control servo, basically is a PWM (Pulse Width Modulation) signal of 50 Hz (or refresh rate: 20ms). It is the width of carrier pulse that decide the position of the torque (not the number of pulse).

Figure D-10 Servo Signal illustration

In order for the servo to keep the position, the signal must be maintain.

In short words, the pulse width of carrier signal will act as a coordinate value of the servo’s torque. Once the coordinate is changed, the servo constantly move the toque to the responding position discarding of its current position. The rotating speed will be depending on the manufacturer of the servo.

For the HG-645MG servo, the corresponding signal is showed below:

Figure D-11 HC 645MG Signal Details

##### Servo’s Speed Control

As we know, the servo will always rotate at a constant speed, for example the  servo HG-645MG speed is 60o/ 0.2s with 6V supply. With such speed applied to robot’s movement, it can affect the stability of the structure, especially for those in which mechanical part is not so much reliable. How do we slow it down?

The idea is not making the servo rotate a whole distance at once but make it rotate one step at a time until it reach its destination, in short it is “divide and conquer” method. For example, you want your HG-645MG servo to rotate 90o in 1 second and the current position of your servo is 0o. If you just apply the PWM with 1500 µs pulse width (1500 µs stand for 90o position), the servo will move to its destination in only 0.3 second. With the method mentioned above, we divide the whole 90o into ten 9o pieces (or 150 µs step as we call it). The signal pulse width will be modified 10 times, after each time the width will be added 150 µs more and waiting 100 ms until the next change happen. With this implementation, the servo will move 10x9o = 90o in 10x100 =  1000 ms = 1 s. The larger the divisor is, the smoother the servo will move.

For this project, the step is always fix at 20 µs and the waiting time fall in range of 10 ms to 20 ms depend on user purposes.

Let D be the travelling distance (in µs), T be the expected time for the move, δ be the waiting time for each step. We have the speed formula:

δ= T ×20D

#### Sensor Reader (HC-SR04)

##### HC-SR04 Specifications

Figure D-12 HC-SR04 Specifications

Figure D-13 HC-SR04 Best practice angle

##### How to read distance value?

The HC-SR04 has 4 pins:  VCC, GND, Trigger, Echo.

- VCC is used for power supply

- GND is used for voltage ground

- Trigger and Echo will perform as follow

Figure D-14 HC-SR04 Operating sequence

First the Trigger pin is pull high for more than 10 µs (10 µs pulse), then waiting for the Echo output to go high and start timing the length of the Echo pin staying high. At this point, a short ultrasonic pulse is transmitted, waiting to be reflected by an obstacle. When the reflected ultrasonic pulse is receive by senor, the Echo output will go low.

The time that Echo output staying high will be used to calculate the distance by following formula given by the manufacturer:

distance cm=pulse width (μs)58

Or

distance inch=pulse width (μs)148

#### Multi-legs Control

The description below is described in the following article by Oricom Technologies – “Analysis of Multi-Legged Animal + Robot Gaits” (2003).

##### Tripod Gait

The tripod gait is the best-known hexapod gait. Hexapod six legs is divide into 2 halves for to control. Each half (or tripod) consists of 3 legs: the front and rear leg of one side and the center leg of the opposite side. More specific:

- Tripod A: left front, right center and left rear

- Tripod B: right front, left center and right rear.

Figure D-15 Tripod Gait illustration

During walking cycle, the weight of the entire hexapod is shifted from one tripod to another. At a moment, there is always one tripod on the ground while another doing its cycle. “Since 3 legs are on the ground at all times, this gait is both "statically" and "dynamically" stable.” - (Oricom Technologies , 2003)

##### Wave Gait

The wave gait implementation make each leg in one side do it cycle after one another leg has finished its cycles and repeat it on the other side’s legs.

Figure D-16 Wave Gait illustration

For instance, the turn of leg taking step is as follow: Left Front -> Left Center -> Left Rear -> Right Front -> Right Center -> Right Rear

Figure D-17 Wave Gait step order

Since there is always 5 legs on the ground, this method is the most stable gait but in return, it is also the slowest gait due to its “one step at a time”.

##### Ripple Gait

“The final stride is the Ripple Gait. At first glance the timing of this gait looks somewhat complicated, however, the key to understanding is to recognize that, on each side a local wave comprising non-overlapping lift phases is being performed, and that the 2 opposite side waves are exactly 180 degrees out of phase with one another. For instance, if L3 and R3 are considered to represent the start of each local wave, then notice that R3 starts to move exactly in the middle of the L3-L2-L1 side wave” .” (Oricom Technologies , 2003)

Figure D-18 Ripple Gait illustration

##### Implementation Approaches

For this project, we will apply the tripod gait method to control the APOD walking movement. As we know, 6 legs of APOD will be divided into 2 tripods

We defined 2 tripod as follow:

- Tripod A: Left front, Right Center, Left Rear

- Tripod B: Right front, Left Center, Right Rear

###### Walking In Straight Line

Leg’s Position:

There are 6 basic positions for the APOD’s leg divive into 2 orientation.

- Vertical: High, Middle , Low

- Horizontal: Front, Center, Rear

Figure D-19 APOD leg position block

Each step (swing time) of the APOD leg is represented by a sequence of 4 consecutive states defined by the position of the leg in both team of vertical and horizontal positions.

For example, this is the forward step sequence:

| State | Vertical | Horizontal |
| --- | --- | --- |
| 0 | Low to Mid | Rear |
| 1 | Mid to High | Rear to Center |
| 2 | High to Mid | Center to Front |
| 3 | Mid to Low | Front |

Table D-1

Figure D-20 APOD leg swing diagram

Depend on the step direction (forward or backward) the sequence may differ: “Front” and “Rear” may be replaing each other’s position.

After making the step, the leg will return to its original position (compare to the body) while keeping the feet on the ground (stand time). This is the point the force was born to push the body moving in the opposite direction of the moving legs.

From the APOD view point, the whole operations of moving leg can be seen as the figure below:

Figure D-21 APOD Leg full step diagram

The whole operation of APOD’s leg can be seen as this sequence of states (Forward step) looping times to times:

| State | Vertical | Horizontal |
| --- | --- | --- |
| 0 | Low | Front to Center |
| 1 | Low | Center to Rear |
| 2 | Low | Rear |
| 3 | Low to Mid | Rear |
| 4 | Mid to High | Rear to Center |
| 5 | High to Mid | Center to Front |
| 6 | Mid to Low | Front |
| 7 | Low | Front |

Table D-2 APOD Leg full forward step

Tripod Position

According to the nature of  The Tripod Gait, every 3 legs in one tripod will always move and stop at the same time. In other words, their cycles are exactly the same phase with one another. The term tripod position is defined by the combination of its legs positions.

Let’s take a view of Tripod A (marked with red color) from the up top viewpoint for example:

- Position: Front (close to head)

Figure D-22 Tripod A at Front position

- Position: Center or Neutral

Figure D-23 Tripod A at Neutral position

- Position: Rear (far from head)

Figure D-24 Tripod A at Rear position

Since all Tripod A’s Legs making the forward step at once (moving from front to rear), there will be 2 pushing force from both side of the body, from the back to the head, that making the APOD moving forward. This is the fundamental of making the APOD moving forward. The backward move can be achieve by making the reverse step of forward step (the backward step).

Walking

Now before we can figure out the walking method of APOD, let’s examine the human walking method first. As we can see, the position of one leg in walking (or running) is always the opposite position of the other one. Or in short, their cycles is exactly 180o out of phase with one another. The similar mechanism is applied for The Tripod Gait walking method. Let Tripod A and B stand for 2 legs of human, then their cycles will always be 180o out of phase with the other one. In conclusion, the full sequence for 2 tripods of The Tripod Gait walking method is described as below

Forward Sequence

| State | Tripod A |  | Tripod B |  |
| --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal |
| 0 | Low | Front to Center | Mid to High | Rear to Center |
| 1 | Low | Center to Rear | High to Mid | Center to Front |
| 2 | Low | Rear | Mid to Low | Front |
| 3 | Low to Mid | Rear | Low | Front |
| 4 | Mid to High | Rear to Center | Low | Front to Center |
| 5 | High to Mid | Center to Front | Low | Center to Rear |
| 6 | Mid to Low | Front | Low | Rear |
| 7 | Low | Front | Low to Mid | Rear |

Table D-3 Forward Sequence

Backward Sequence

| State | Tripod A |  | Tripod B |  |
| --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal |
| 0 | Low | Rear to Center | Mid to High | Front to Center |
| 1 | Low | Center to Front | High to Mid | Center to Rear |
| 2 | Low | Front | Mid to Low | Rear |
| 3 | Low to Mid | Front | Low | Rear |
| 4 | Mid to High | Front to Center | Low | Rear to Center |
| 5 | High to Mid | Center to Rear | Low | Center to Front |
| 6 | Mid to Low | Rear | Low | Front |
| 7 | Low | Rear | Low to Mid | Front |

Table D-4 Backward Sequence

###### Turning Left – Right

As mention above, we already know 3 basic position of tripod: Front, Center, Rear. In this section, there are 2 more position: Left and Right.

The definition  of 2 new position can be described by tripod A as follow:

- Position: Left

Figure D-25 Tripod A at Left position

- Position: Right

Figure D-26 Tripod A at Right position

Let’s take a look when Tripod A making set :

- Left Front and Left Rear take the forward step

- Right Center take the backward step

There will be forces with 2 opposite direction on the APOD body. The left side force push the body forward and the right side force push the body backward. As a result of this combination. The APOD will turn from left to right. In other words, when the tripod making transition from left position to right position (through center position), the APOD will turn from left to right. And when the tripod making transition from right position to left position (through center position), the APOD will turn from right to left.

The combination sequence for turning the APOD:

Turn Left:

| State | Tripod A |  | Tripod B |  |
| --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal |
| 0 | Low | Left to Center | Mid to High | Right to Center |
| 1 | Low | Center to Right | High to Mid | Center to Left |
| 2 | Low | Right | Mid to Low | Left |
| 3 | Low to Mid | Right | Low | Left |
| 4 | Mid to High | Right to Center | Low | Left to Center |
| 5 | High to Mid | Center to Left | Low | Center to Right |
| 6 | Mid to Low | Left | Low | Right |
| 7 | Low | Left | Low to Mid | Right |

Table D-5 Turn Left Sequence

Turn Right:

| State | Tripod A |  | Tripod B |  |
| --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal |
| 0 | Low | Right to Center | Mid to High | Left to Center |
| 1 | Low | Center to Left | High to Mid | Center to Right |
| 2 | Low | Left | Mid to Low | Right |
| 3 | Low to Mid | Left | Low | Right |
| 4 | Mid to High | Left to Center | Low | Right to Center |
| 5 | High to Mid | Center to Right | Low | Center to Left |
| 6 | Mid to Low | Right | Low | Left |
| 7 | Low | Right | Low to Mid | Left |

Table D-6 Turn Right Sequence

### SSC32 Board

#### Schematic

Figure D-27 SSC32 Schematic

#### Technical Point

##### Atmega328P Microcontroller (MCU)

###### Overview

ATmega328p is a high-performance Atmel picoPower 8-bit AVR RISC-based (Reduced Instruction Set Computing) microcontroller provided with these features:

- 32KB ISP (In-System Programming) flash memory with read-while-write capabilities

- 1024B EEPROM (Electrically Erasable Programmable Read-Only Memory)

- 2KB SRAM (Static Random-Access Memory)

- 23 general purpose I/O lines

- 32 general purpose working registers

- Three flexible timer/counters with compare modes

- Internal and external interrupts

- Serial programmable USART (Universal Synchronous and Asynchronous serial Receiver and Transmitter)

- A byte-oriented 2-wire serial interface

- SPI (Serial Peripheral Interface) serial port

- A 6-channel 10-bit A/D

- Programmable watchdog timer with internal oscillator

- Five software selectable power saving modes

Figure D-28 ATmega328p Microcontroller

For more information about ATmega328 Features and Specifications, please refer to the ATmega328p Datasheet from Atmel web site:

http://www.atmel.com/devices/atmega328p.aspx?tab=documents

###### Pin Configuration

Figure D-29 ATmega328p 28 Pins DIP

###### Block Diagram

Figure D-30 ATmega328p Block Diagram

##### General Purposes Input / Output (GPIO)

The ATmega328p has 3 Ports: Port B, Port C, Port D and each of them consists of 8 pin PB[0..7], PC[0..7], PD[0..7]

Each port pin consists of three register bits: DDxn, PORTxn, and PINxn. The DDxn bits are accessed at the DDRx I/O address, the PORTxn bits at the PORTx I/O address, and the PINxn bits at the PINx I/O address. The details will be described later.

Notes:

- “x” represent the port name, can be “B”,”C” or “D”.

- “n” represent the pin number of the x port, vary from 0 to 7 (each port has 8 pins, as mentioned above)

###### Configurations

The DDxn bit in the DDRx Register selects the direction of this pin. If DDxn is written logic one, Pxn is configured as an output pin. If DDxn is written logic zero, Pxn is configured as an input pin.

If PORTxn is written logic one when the pin is configured as an input pin, the pull-up resistor is activated. To switch the pull-up resistor off, PORTxn has to be written logic zero or the pin has to be configured as an output pin.

If PORTxn is written logic one when the pin is configured as an output pin, the port pin is driven high (one). If PORTxn is written logic zero when the pin is configured as an output pin, the port pin is driven low (zero).

Writing a logic one to PINxn toggles the value of PORTxn, independent on the value of DDRxn.

###### Register Description

MCUCR

Figure D-31 MCU Control Register

- Bit 4 - PUD: Pull-up Disable

When this bit is written to one, the pull-ups in the I/O ports are disabled even if the DDxn and PORTxn Registers are configured to enable the pull-ups.

PORT B

Figure D-32 PORT B Register Description

PORT C

Figure D-33 PORT C Register Description

PORT D

Figure D-34 PORT D Register Description

##### Interrupts

The ATmega328p provides 26 interrupt vectors for user. In the scope of this project, we only use some of the interrupts services listed in the table below:

| Number | Source | Definition |
| --- | --- | --- |
| 1 | PCINT1 | Pin Change Interrupt Request 1 |
| 2 | TIMER1 COMPA | Timer/Counter1 Compare Match A |
| 3 | TIMER1 COMPB | Timer/Coutner1 Compare Match B |
| 4 | TIMER0 COMPA | Timer/Counter0 Compare Match A |
| 5 | USART, RX | USART Rx Complete |
| 6 | USART, TX | USART, Tx Complete |

Table D-7 Interrupts Usage

##### Timer

###### Usage and Purposes

The main feature of the SSC32 board is to control servos, more specific: 32 HC645MG servos. As mention in previous section, a RC servo is controlled by a signal, which basically is a PWM signal or simply an output pin toggled at specific time (high <-> low). It is the responsibility of Timer making the toggle to generate the PWM signal controlling servos. To support that purpose of this project, we will use 16-bit Timer of the ATmega328p (TIMER/COUNTER 1).

###### Register Description

TTCR1A

Figure D-35 Timer 1 Control register A

- Bit 7:6 - COM1A1:0: Compare Output Mode for Channel A

- Bit 5:4 - COM1B1:0: Compare Output Mode for Channel B

- Bit 3:2 - Reserved: read as “0”

- Bit 1:0 - WGM11:0: Waveform Generation Mode

TTCR1B

Figure D-36 Timer 1 Control register B

- Bit 7 - ICNC1: Input Capture Noise Canceler

- Bit 6 - ICES1: Input Capture Edge Select

- Bit 5 - Reserved: Read as “0”

- Bit 4:3 - WGM13:2: Waveform Generation Mode

- Bit 2:0 - CS12:0: Clock Select

TTCR1C

Figure D-37 Timer 1 Control register C

- Bit 7 - FOC1A: Force Output Compare for Channel A

- Bit 6 - FOC1B: Force Output Compare for Channel B

- Bit 5:0 - Reserved: Read as “0”

TCNT1

Figure D-38 Timer 1 Counter

ORC1A

Figure D-39 Output Compare Register A

ORC1B

Figure D-40 Output Compare Register B

IRC1

Figure D-41 Input Capture Register

TIMSK1

Figure D-42 Timer/Counter1 Interrupt Mask Register

- Bit 7, 6 - Reserved: Read as “0”

- Bit 5 - ICIE1: Timer/Counter1, Input Capture Interrupt Enable

- Bit 4, 3 - Reserved: Read as “0”

- Bit 2 - OCIE1B: Timer/Counter1, Output Compare B Match Interrupt Enable

- Bit 1 - OCIE1A: Timer/Counter1, Output Compare A Match Interrupt Enable

- Bit 0 -TOIE1: Timer/Counter1, Overflow Interrupt Enable

TIFR1

Figure D-43 Timer 1 Interrupt Flog register

- Bit 7, 6 - Reserved: Read as “0”

- Bit 5 - ICF1: Timer/Counter1, Input Capture Flag

- Bit 4, 3 - Reserved: Read as “0”

- Bit 2 - OCF1B: Timer/Counter1, Output Compare B Match Flag

- Bit 1 - OCF1A: Timer/Counter1, Output Compare A Match Flag

- Bit 0 - TOV1: Timer/Counter1, Overflow Flag

##### Universal Synchronous and Asynchronous serial Receiver and Transmitter (USART)

###### Usage and Purposes

In this project, the use of USART will be the main communication channel between SSC32 module and other modules. The command will be receive through this channel as well as the response from SSC32 (when required).

###### Baud Rate

The SSC32 support method for user to change baud rate of USART via 2 jumper on board baud0 and baud1

Figure D-44 Baud Rate setup

| BAUD0 | BAUD1 | Baud Rate | UBRR0H | UBRR0L |
| --- | --- | --- | --- | --- |
| 0 | 0 | 2400 | 1 | 7F |
| 0 | 1 | 9600 | 0 | 5F |
| 1 | 0 | 38400 | 0 | 17 |
| 1 | 1 | 115200 | 0 | 7 |
| Note: - 1: indicate jumper is close - 0: indicate jumper open - The details about UBRR0H and UBRR1H is in the section below. |  |  |  |  |

Table D-8 Baud Rate setup details

After apply the jumper(s) on baud rate selection pin(s), the ATmega328p must be reset for the new baud rate to be applied.

###### Register Description

UDRn

Figure D-45 USART I/O Data register

The USART Transmit Data Buffer Register and USART Receive Data Buffer Registers share the same I/O address referred to as USART Data Register or UDRn. The Transmit Data Buffer Register (TXB) will be the destination for data written to the UDRn Register location. Reading the UDRn Register location will return the contents of the Receive Data Buffer Register (RXB).

UCSRnA

Figure D-46USART Control and Status Register A

- Bit 7 - RXCn: USART Receive Complete

- Bit 6 - TXCn: USART Transmit Complete

- Bit 5 - UDREn: USART Data Register Empty

- Bit 4 - FEn: Frame Error

- Bit 3 - DORn: Data OverRun

- Bit 2 - UPEn: USART Parity Error

- Bit 1 - U2Xn: Double the USART Transmission Speed

- Bit 0 - MPCMn: Multi-processor Communication Mode

UCSRnB

Figure D-47 USART Control and Status register B

- Bit 7 - RXCIEn: RX Complete Interrupt Enable n

- Bit 6 - TXCIEn: TX Complete Interrupt Enable n

- Bit 5 - UDRIEn: USART Data Register Empty Interrupt Enable n

- Bit 4 - RXENn: Receiver Enable n

- Bit 3 - TXENn: Transmitter Enable n

- Bit 2 - UCSZn2: Character Size n

- Bit 1 - RXB8n: Receive Data Bit 8 n

- Bit 0 - TXB8n: Transmit Data Bit 8 n

UCSRnC

Figure D-48 USART Control and Status register C

- Bits 7:6 - UMSELn1:0 USART Mode Select

- Bits 5:4 - UPMn1:0: Parity Mode

- Bit 3 - USBSn: Stop Bit Select

- Bit 2:1 - UCSZn1:0: Character Size

- Bit 0 - UCPOLn: Clock Polarity

UBRRnL and UBRRnH

Figure D-49 USART Baud Rate register

- Bit 15:12 - Reserved: read as “0”

- Bit 11:0 - UBRR[11:0]: USART Baud Rate Register

Figure D-50 Baud Rate Calculation

##### Serial Peripheral Interface (SPI)

###### Usage and Purposes

In this project, SPI communication will only be used for communication between ATmega328P and 74HC595, not for communication between microcontrollers. The detail of SPI- Shift Register interface will be discussed later in this document.

###### Register Description

SPCR

Figure D-51 SPI Control register

- Bit 7 - SPIE: SPI Interrupt Enable

- Bit 6 - SPE: SPI Enable

- Bit 5 - DORD: Data Order

- Bit 4 - MSTR: Master/Slave Select

- Bit 3 - CPOL: Clock Polarity

- Bit 2 - CPHA: Clock Phase

- Bits 1, 0 - SPR1, SPR0: SPI Clock Rate Select 1 and 0

SPSR

Figure D-52 SPI Status register

- Bit 7 - SPIF: SPI Interrupt Flag

- Bit 6 - WCOL: Write Collision Flag

- Bit [5:1] – Reserved: Read as “0”

- Bit 0 - SPI2X: Double SPI Speed Bit

SPDR

Figure D-53 SPI Data register

The SPI Data Register is a read/write register used for data transfer between the Register File and the SPI Shift Register

##### Shift Register (74HC595)

###### Overview

74HC595 is a shift register device with 8-bit serial input 8-bit serial or parallel output. This project will use the “serial to parallel conversion” feature of this Shift register.

###### How does it work?

Figure D-54 74HC595 Block diagram

- Serial Data Input (DS): This pin is connected to any port of the microcontroller. This pin is responsible for getting the data serially from the microcontroller.

- Shift Clock (SH_CP): On this pin the clock signal is applied. On the rising edge, that is when the clock rises from negative to positive, the data on the DS line is sampled and it gets stored in the shift register. The bit on DS line is stored on the LSB (least significant place i.e. BIT0). On next pulse this bit moves to BIT1 location. After 8 clock pulses this bit is moved to BIT7 (MSB place). After 8 clock pulses the Shift Register has all 8 bits of a byte and is ready to convert them to parallel.

- Store Clock (ST_CP): You can hold this pin LOW while you get everything setup and nothing on the display pins will change. Then when you are done, and everything is how you want, you pull the pin HIGH and the 74HC595 will display the new settings. So even though we are changing values in the register in 8 steps, it looks like it was just one step

- MR : – Will empty the whole shift register, if pulled low, must be pulled high to enable.

- OE: – This pin enables the output when tied to ground, & disabled when HIGH.

##### SPI - Shift Register interface

###### Overview

Let’s take a look of how we generate the signal controlling servos using timer. We have 32 servos to be control by the SSC32 Board.

First approach, we will use Output compare feature of timer to generate the PWM signal. With this built-in feature, we can generate highly accurate PWM signal. But the problem is that ATmega328p have only 2 output compare pin for each timer. Even if we use all 3 timers, we can only control 6 servos by this approach.

Second approach, we will use the GPIO to output the PWM signal. The PWM will be toggle manually for each pins in ports. But on total, ATmega328 has only 3 GPIO ports which mean we can only control 8x3 = 24 servos while having 32 servos to control and also wasting all the other features of the MCU.

The third approach is using some external device. In this case, we will use 4 more Shift Registers (74HC595) to output 32 PWM signal (8 pin on each Shift Register).

###### How does it work?

Take a look on SPI diagram

Figure D-55 SPI Diagram

We know that after the SPI Data register is written the SPI transmitting protocol will be trigger to send out data.

Data in the SPI Data register will be shifted out one bit at a time on the SCK clock, start with the MSB (Most Significant Bit or Most Left Bit). At one clock of SCK, one bit of data will be shift out on MOSI (Master Out Slaver In) pin.

Now we wire the 74HC595 as follow:

Figure D-56 SPI- Shift Register

Ignore the MISO (Master In Slaver Out) pin, the MOSI pin is now connect to the DS (Or SDI) pin of the Shift register. SCK will be connected to the SH_CP Pin of the Shift register (or also SCK).

When the SPI transmitting is trigger, the SPI clock will also be the Shifting clock on the Shift Register. Each bit of the SPI data will follow the clock transfer into the 74HC595 Register through the MOSI – SDI wire. As a result, the SPI become a tool for writing data to shift register automatically. The output of 74HC595 can only change when a high pulse is apply to the ST_CP Pin (or the RCK as the Figure above).

Using the same MISO and SCK pin for 4 Shifter, 4 more pin for the output trigger. We have extended 6 pins of ATmega328p into 32 pins, which is enough for controlling 32 servos as mentioned above, while not wasting other feature of the ATmega328p.

### STM32F4 Discovery Board

## ACTIVITY AND CLASS STRUCTURE

### Activity

#### APOD Controller (HMI Application)

Figure D-57

#### STM32F4 Discovery (Central processing board)

Figure D-58

#### SSC32 Board (Servos control and Sensor reader)

Figure D-59

### Structure

#### APOD Controller (HMI Application)

#### Central Processing Module (STM32F4)

#### Servo Control & Sensor Reader Module (SSC32)

## SEQUENCE DIAGRAM

### Embedded Controller

#### Forward

Figure D-61

#### Backward

Figure D-62

#### Turn Left

Figure D-63

#### Turn Right

Figure D-64

### Use-cases Sequences

#### Sequences Control

##### Add State

Figure D-65

##### Remove States

Figure D-66

##### Import States

Figure D-67

##### Export States

Figure D-68

#### Direct Control

##### Mode Selection

###### Normal Input

Figure D-69

###### Gamepad Input

Figure D-70

##### Movement Control

###### Forward

Figure D-71

###### Backward

Figure D-72

###### Turn Left

Figure D-73

###### Turn Right

Figure D-74

###### Body Lift

Figure D-75

###### Body Drop

Figure D-76

###### Squeeze Left

Figure D-77

###### Squeeze Right

Figure D-78

#### Object Tracking

##### Auto Grip

Figure D-79

##### Choose target

Figure D-80

##### Change Target

Figure D-81

##### Lock Target

Figure D-82

## USER INTERFACE DESIGN

### Live Control

4

2

3

10

13

12

14

11

8

9

7

6

5

1

0

Figure D-83 Live Control Interface Design

| Item NO. | Component Name | Description |
| --- | --- | --- |
| 0 | Menu | Select Features |
| 1 | VideoSourcePlayer | Display Camera IP ‘s View |
| 2 | Checkbox (Button) | Select Normal input mode |
| 3 | Checkbox (Button) | Select Gamepad input mode |
| 4 | Checkbox (Button) | Select object tracking mode |
| 5 | Button | Start streaming video from Camera |
| 6 | Image | Display tracking template |
| 7 | Button | Change Tracking template |
| 8 | Checkbox (Button) | Lock target – Start tracking |
| 9 | Button | Switch speed |
| 10 | Button | Auto grip |
| 11 | Keypad | Action selection |
| 12 | Keypad | Action bank selection |
| 13 | Keypad | Set mode/function |
| 14 | Keypad | Navigation Selection |

Table D-9 Live Control Interface Design Description

### Sequence Player

4

7

6

5

3

1

2

0

Figure D-84 Sequence Player Interface Design

| Item NO. | Component Name | Description |
| --- | --- | --- |
| 0 | Combo Box | Select action to add |
| 1 | Textbox | Movement interval |
| 2 | Button | Add action to sequence |
| 3 | Data Grid | Display current sequence content |
| 4 | Button | Remove selected states from sequence |
| 5 | Button | Export current sequence to file |
| 6 | Button | Import sequence from file |
| 7 | Button | Start playing the sequence |

Table D-10 Sequence Player Interface Design Description

### Configuration

85

75

65

5

44

35

25

1

0

Figure D-85 Configuration Interface Design

| Item NO. | Component Name | Description |
| --- | --- | --- |
| 0 | Textbox | Enter IP of camera |
| 1 | Textbox | Camera port to get stream |
| 2 | Textbox | Username of camera IP |
| 3 | Password Box | Password of camera IP |
| 4 | Combo Box | All available COM ports |
| 5 | Combo Box | Bluetooth baudrate selection |
| 6 | Button | Refresh list of Ports |
| 7 | Button | Accept information |
| 8 | Button | Cancel action |

Table D-11 Configuration Interface Design Description

### Object Extractor

25

1

0

Figure D-86 Object Extractor Interface Design

| Item NO. | Component Name | Description |
| --- | --- | --- |
| 0 | Image | Capture frame for template selection |
| 1 | Button | Accept new template |
| 2 | Button | Cancel action |

Table D-12 Object Extractor Interface Design Description

# SOFTWARE TEST DOCUMENTATION (STD)

## Introduction

### Overview

The purpose of this project is to build a physical hexapod robotic with named A-pod. He has the ability to get balance on his legs, move  like a real insect, can record by the camera and auto follow and grip item which is chosen. In the other hand, the robot communicate with Pc through Bluetooth technology, and be Controlled by PS2 controller

### Test Approach

White box testing : Developers self-test on code in which function they developed (unit test).

Black box testing : Test on each function of each module in system to ensure each module execute the right function. Then do integration testing to test the function. Finally, system test to test the whole system.

Testing Goal : Discover bug , fix code, regression test and finally completed system.

## Test Plan

We have two main kind of test phases: Function test (Integration test) and System test.

For the Integration test, we get the functional test to show up the working of each function is right or wrong, so we have to focus on the result of the function in difference cases.

For the System test, all activities of system are needed to check after setup, for example what does he(A-pod) do when we start up and observe him if something happen . We create test case to check each activity of him from turn on to turn off and log what he do when we act on. Finally, we record the result

### Features to be tested

Functional Testing:

- Integration Testing: Test all function of robot, C# application

- System Testing: Test all function for APOD – C# application

UI Testing:

- Test UI of C# application

### Features not to be tested

Embedded function on STM32F4 and SSC32

### Test Environment

In addition to the application and any other user specified software, the following list of software should be considered a minimum:

- NET Framework 4.0

- Visual Studio 2012

Hardware tools:

- A PC/laptop that has Bluetooth device and can connect Wireless.

- Wireless Router

Operating System: Window 8

## Test Cases

### Functional Test

| No | Function |  | Pre-condition | Step of Testing | Expected result | Create by | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Medium Function | Small Function |  |  |  |  |  |
| 1 | Start robot | Turn robot on | Robot is turned off | Switch the trigger on robot | Notice led on two board is on | QuanNM | P |
| 2 |  | Turn robot off | Robot is turned on | Switch the trigger on robot | Notice led on two board is off | QuanNM | P |
| 3 | Connect Bluetooth device | Connect PC/Laptop that has Bluetooth | Robot is turned on | 1.Turn ON Bluetooth board on two device 2.Pair device and enter pin ’1234’ on PC | 1. If two devices can connect to each other, the light of Bluetooth board on Robot is continuous light. If two devices can't connect to each other, the light of Bluetooth board on Robot is flashing light. | QuanNM |  |
| 4 | Operate in “Gamepad Controller” mode | Start robot | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 | Click “Start” button on PS2 controller | 1.Robot start 2. All servos are set with 1500 pulsewidth | QuanNM | P |
| 5 |  | Stop robot | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Select] button on PS2 controller | 1. Robot is stop. 2. All servos are released. | QuanNM | P |
| 6 |  | Robot move forward | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Up Dpad] button on PS2 controller | Robot move forward 1 time | QuanNM | P |
| 7 |  | Robot move backward | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Down Dpad] button on PS2 controller | Robot move backward 1 time | QuanNM | P |
| 8 |  | Turn robot left | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Left Dpad] button on PS2 controller | Robot turn left 1 time | QuanNM | P |
| 9 |  | Turn robot right | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Right Dpad] button on PS2 controller | Robot turn right 1 time | QuanNM | P |
| 10 |  | Drop robot’s body | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Trigger R1] on PS2 controller | Robot drop the height 1 time | QuanNM | P |
| 11 |  | Lift Robot’s body | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Trigger R2] on PS2 controller | Robot lift the height 1 time | QuanNM | P |
| 12 |  | Stretch robot forward | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Up Dpad] on PS2 controller | Robot stretch forward 1 time | QuanNM | P |
| 13 |  | Stretch robot backward | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Down Dpad] on PS2 controller | Robot stretch backward 1 time | QuanNM | P |
| 14 |  | Stretch robot left | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Left Dpad] on PS2 controller | Robot stretch to left 1 time | QuanNM | P |
| 15 |  | Stretch robot right | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Right Dpad] on PS2 controller | Robot stretch to right 1 time | QuanNM | P |
| 16 |  | Rotate Robot’s neck to left | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Square] on PS2 controller | Robot’s neck rotate to left 1 time | QuanNM | P |
| 17 |  | Rotate Robot’s neck to right | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Circle] on PS2 controller | Robot’s neck rotate to right 1 time | QuanNM | P |
| 18 |  | Turn robot’s neck up | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L2] + [Up Dpad] on PS2 controller | Robot’s neck turn up 1 time | QuanNM | P |
| 19 |  | Turn robot’s neck down | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click combo button [Trigger L2] + [Down Dpad] on PS2 controller | Robot’s neck turn down 1 time | QuanNM | P |
| 20 |  | Turn robot’s neck left | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click combo button [Trigger L2] + [Left Dpad] on PS2 controller | Robot’s neck turn left 1 time | QuanNM | P |
| 21 |  | Grip robot’s ripper | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Triangle] button on PS2 controller | Robot’s Gripper grip 1 time | QuanNM | P |
| 22 |  | Release robot’s ripper | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click combo button [Trigger L2] + [Left Dpad] on PS2 controller | Robot’s Gripper release 1 time | QuanNM | p |
| 23 | Operate in “Normal” mode (using keyboard) | Start robot | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 | Click “Start” button on Main PC Interface | 1.Robot start 2. All servos are set with 1500 pulse width | QuanNM | P |
| 24 |  | Stop robot | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Select] button on Main PC Interface | 1. Robot is stop. 2. All servos are released. | QuanNM | P |
| 25 |  | Robot move forward | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Up Dpad] button on Main PC Interface | Robot move forward 1 time | QuanNM | P |
| 26 |  | Robot move backward | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Down Dpad] button Main PC Interface | Robot move backward 1 time | QuanNM | P |
| 27 |  | Turn robot left | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Left Dpad] button Main PC Interface | Robot turn left 1 time | QuanNM | P |
| 28 |  | Turn robot right | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is turned on | Click [Right Dpad] button Main PC Interface | Robot turn right 1 time | QuanNM | p |
| 29 |  | Drop robot’s body | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Trigger R1] Main PC Interface | Robot drop the height 1 time | QuanNM | P |
| 30 |  | Lift Robot’s body | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Trigger R2] Main PC Interface | Robot lift the height 1 time | QuanNM | P |
| 31 |  | Stretch robot forward | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Up Dpad] Main PC Interface | Robot stretch forward 1 time | QuanNM | P |
| 32 |  | Stretch robot backward | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Down Dpad] Main PC Interface | Robot stretch backward 1 time | QuanNM | P |
| 33 |  | Stretch robot left | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Left Dpad] on Main PC Interface | Robot stretch to left 1 time | QuanNM | P |
| 34 |  | Stretch robot right | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L1 ] + [Right Dpad] on Main PC Interface | Robot stretch to right 1 time | QuanNM | P |
| 35 |  | Rotate Robot’s neck to left | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Square] on Main PC Interface | Robot’s neck rotate to left 1 time | QuanNM | P |
| 36 |  | Rotate Robot’s neck to right | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Circle] on Main PC Interface | Robot’s neck rotate to right 1 time | QuanNM | P |
| 37 |  | Turn robot’s neck up | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click Combo button [Trigger L2] + [Up Dpad] on Main PC Interface | Robot’s neck turn up 1 time | QuanNM | P |
| 38 |  | Turn robot’s neck down | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click combo button [Trigger L2] + [Down Dpad] on Main PC Interface | Robot’s neck turn down 1 time | QuanNM | P |
| 39 |  | Turn robot’s neck left | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click combo button [Trigger L2] + [Left Dpad] on Main PC Interface | Robot’s neck turn left 1 time | QuanNM | P |
| 40 |  | Grip robot’s ripper | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click [Triangle] button on Main PC Interface | Robot’s Gripper grip 1 time | QuanNM | P |
| 41 |  | Release robot’s ripper | 1. Robot and PC/ Laptop connected with each other 2. Environment: - Using Serial Port communication in win 7/8 3. Robot is on | Click combo button [Trigger L2] + [Left Dpad] on Main PC Interface | Robot’s Gripper release 1 time | QuanNM | P |

### UI Test

| No | Function |  | Pre-condition | Step of Testing | Expected result | Create by | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Medium Function | Small Function |  |  |  |  |  |
| 1 | Normal mode | Connect Bluetooth with normal controller Connect Bluetooth with normal controller | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera:”192.168.2.100” - Port: “80” - Admin: “admin” - Password: NONE - Com port: COM3 (depend on each computer) -Baud rate: “9600” 3. Click OK | - Configuration windows is disappear. -IP camera connected - Bluetooth pair with robot | QuanNM | P |
| 2 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera: NONE - Port: NONE - Admin: “admin” - Password: NONE - Com port: NONE(depend on each computer) -Baud rate: “9600” 3. Click OK | 1.Each blank detail( IP camera, Port, Com port) will noticed by change the color (red) 2. System still not connect | QuanNM | P |
| 3 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera: NONE - Port: 80 - Admin: “admin” - Password: NONE - Com port: COM No.(depend on each computer) -Baud rate: “9600” 3. Click OK | 1. IP camera is noticed to red color. 2.System still not connect | QuanNM | p |
| 4 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera: “192.168.2.100” - Port: NONE - Admin: “admin” - Password: NONE - Com port: COM No.(depend on each computer) -Baud rate: “9600” 3. Click OK | 1. Port is noticed to red color. 2.System still not connect | QuanNM | P |
| 5 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera: “192.168.2.100” - Port: 80 - Admin: “admin” - Password: NONE - Com port: NONE No.(depend on each computer) -Baud rate: “9600” 3. Click OK | 1. COM Port is noticed to red color. 2.System still not connect | QuanNM | P |
| 6 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera:”192.168.2.100” - Port: “80” - Admin: “admin” - Password: NONE - Com port: COM3 (depend on each computer) -Baud rate: “9600” 3. Click OK | Configuration windows is disappear. | QuanNM | p |
| 7 |  | Connect Bluetooth with normal controller Connect Bluetooth with Keypad controller | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera:”192.168.2.100” - Port: “80” - Admin: “admin” - Password: NONE - Com port: COM3 (depend on each computer) -Baud rate: “9600” 3. Click OK | - Configuration windows is disappear. -IP camera connected - Bluetooth pair with robot | QuanNM | P |
| 8 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera: NONE - Port: NONE - Admin: “admin” - Password: NONE - Com port: NONE(depend on each computer) -Baud rate: “9600” 3. Click OK | 1.Each blank detail( Ip camera, Port, Com port) will noticed by change the color (red) 2. System still not connect | QuanNM | P |
| 9 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera: NONE - Port: 80 - Admin: “admin” - Password: NONE - Com port: COM No.(depend on each computer) -Baud rate: “9600” 3. Click OK | 1. IP camera is noticed to red color. 2.System still not connect | QuanNM | p |
| 10 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera: “192.168.2.100” - Port: NONE - Admin: “admin” - Password: NONE - Com port: COM No.(depend on each computer) -Baud rate: “9600” 3. Click OK | 1. Port is noticed to red color. 2.System still not connect | QuanNM | P |
| 11 |  |  | 1.Application is open. 2.Bluetooth is already to pair 3.Wireless connected | 1. Click “Normal” button. 2.In Configuration, Fill in: - IP camera: “192.168.2.100” - Port: 80 - Admin: “admin” - Password: NONE - Com port: NONE No.(depend on each computer) -Baud rate: “9600” 3. Click OK | 1. COM Port is noticed to red color. 2.System still not connect | QuanNM | P |
| 12 |  | Start camera | 1.Bluetooth is already connected. 2.Wireless is already connected. 3.Camera is already connected. | Click “Start Video” button | Camera screen appear in camera view frame | QuanNM | P |
| 13 | Object Tracking | Getting item | 1.Bluetooth is already connected. 2.Wireless is already connected. 3.Camera is already connected. 4.Camera screen started | 1. Click “Object tracking” button. 2. Click on item’s color 3. Click Ok | 1. Yellow square focus on chosen item. 2. On the right screen there is a show of item’s color | QuanNM | P |
| 14 |  | Follow particular Item | 1.Bluetooth is already connected. 2.Wireless is already connected. 3.Camera is already connected. 4.Camera screen started 5. Item is tracked | Click “Auto” button | 1.On APOD: - Gripper is open widely. - APOD auto moves to item. - After he reach the item, then he nips. 2. On Pc: - Tracking zone is always focus depend on APOD movement | QuanNM | P |
| 15 |  | Get distance | 1.Bluetooth is already connected. 2.Wireless is already connected. 3.Camera is already connected. 4.Camera screen started 5. Item is tracked | Click “Distance” button | Receive current distance from APOD to item. | QuanNM | P |
| 16 | Sequence Player | Making sequence to perform(1) | 1.Bluetooth is already connected. 2.Wireless is already connected. | 1.Click menu “Tool”, choose “Sequence player” menu. 2. In move: chose [Forward]. 3. In Time: Enter [5]. 4. click Add. 5.click Play. | APOD move forward in 5 second | QuanNM | P |
| 17 |  | Making sequence to perform(2) | 1.Bluetooth is already connected. 2.Wireless is already connected. | 1.Click menu “Tool”, choose “Sequence player” menu. 2. In move: chose [Forward]. 3. In Time: Enter [5]. 4. click Add. 5. In move: chose [Turn Left]. 6. In Time: Enter [3]. 7.click Play. | APOD move forward in 5 seconds, turn left in 3 seconds | QuanNM | P |
| 18 |  | Making sequence to perform(3) | 1.Bluetooth is already connected. 2.Wireless is already connected. | 1.Click menu “Tool”, choose “Sequence player” menu. 2. In move: chose [Forward]. 3. In Time: Enter [5]. 4. click Add. 5. In move: chose [Turn Left]. 6. In Time: Enter [3]. 7. In move: chose [Turn Right]. 8. In Time: Enter [10]. 9.click Play. | APOD move forward in 5 seconds, turn left in 3 seconds, turn right in 10 seconds | QuanNM | P |
| 19 |  | Making sequence to perform(4) | 1.Bluetooth is already connected. 2.Wireless is already connected. | 1.Click menu “Tool”, choose “Sequence player” menu. 2. In move: chose [Backward]. 3. In Time: Enter [5]. 4. click Add. 5. In move: chose [Head left]. 7. In move: chose [toward Front]. 8.click Play. | APOD move forward in 5 seconds, Head to left, APOD is toward front | QuanNM | P |
| 20 |  | Making sequence to perform(4) | 1.Bluetooth is already connected. 2.Wireless is already connected. | 1.Click menu “Tool”, choose “Sequence player” menu. 2. In move: chose [Forward]. 3. In Time: Enter [-5]. | System prevent to enter “-” | QuanNM | P |
| 21 |  | Export sequence | 1.Bluetooth is already connected. 2.Wireless is already connected. | 1.Click menu “Tool”, choose “Sequence player” menu. 2. In move: chose [Backward]. 3. In Time: Enter [5]. 4. click Add. 5. In move: chose [Head left]. 7. In move: chose [toward Front]. 8.click “Export” button. 9.Choose direction. 10. Enter name of file 11. click OK | Sequence file is created with xml type | QuanNM | P |
| 22 |  | Import sequence | 1.Bluetooth is already connected. 2.Wireless is already connected. | 1.Click Import button 2.chose file xml to import. 3.Click OK. | List of sequence in file are listed to perform | QuanNM | P |
| 23 |  | Remove sequence | 1.Bluetooth is already connected. 2.Wireless is already connected. | 1.Click menu “Tool”, choose “Sequence player” menu. 2. In move: chose [Backward]. 3. In Time: Enter [5]. 4. click Add. 5. In move: chose [Head left]. 7. In move: chose [toward Front]. 8. stick on [Head Left] combo box 8.click Remove button. | [Head left] is removed in list | QuanNM | p |

# SOFTWARE USER MANUAL (SUM)

# APPENDIX

## Glossary

## References

All you need to know about AVR fuses . (2013, May 30). Retrieved from Embedds: http://www.embedds.com/all-you-need-to-know-about-avr-fuses/

Atmel. (n.d.). ATmega328p. Retrieved from Atmel Website: http://www.atmel.com/devices/atmega328p.aspx?tab=overview

Atmel. (n.d.). ATmega328p. Retrieved from Atmel Web site: http://www.atmel.com/devices/atmega328.aspx?tab=documents

Electrical and Computer Engineering. (n.d.). C Coding Standard . Retrieved from Electrical and Computer Engineering Web site: http://users.ece.cmu.edu/~eno/coding/CCodingStandard.html

Frye, J. (2010, June 16). SSC-32 Manual. Retrieved from Lynxmotion Web site: http://www.lynxmotion.com/images/html/build136.htm

ITead Studio. (n.d.). Ultrasonic Ranging Module HC-SR04. Retrieved from ITead Studio Web site: http://imall.iteadstudio.com/im120628012.html

Kirillov, A. (2010, June 30). Detecting some simple shapes in images. Retrieved from http://www.aforgenet.com: http://www.aforgenet.com/articles/shape_checker/

Kirillov, A. (2010, November 5). Glyphs' recognition. Retrieved from http://www.aforgenet.com: http://www.aforgenet.com/articles/glyph_recognition/

Lötzsch, W. (2012, December 9). Visual Studio 2012 Metro Styles for WPF. Retrieved from Codeproject Web site: http://www.codeproject.com/Articles/442856/Visual-Studio-2012-Metro-Styles-for-WPF

NXP . (n.d.). 74HC595PW. Retrieved from NXP Web site: http://www.nxp.com/products/logic/shift_registers/74HC595PW.html

Oricom Technologies . (2003, January). Analysis of Multi-Legged Animal + Robot Gaits. Retrieved from Oricom Technologies Web site: http://www.oricomtech.com/projects/leg-time.htm#Hex2

STMicroelectronics . (n.d.). STM32F4DISCOVERY. Retrieved from STMicroelectronics Web site: http://www.st.com/web/catalog/tools/FM116/SC959/SS1532/PF252419#

STMicroelectronics. (n.d.). STM32F407VG. Retrieved from STMicroelectronics Web site: http://www.st.com/web/catalog/mmc/FM141/SC1169/SS1577/LN11/PF252140#

STMicroelectronics STM32F407VG. (n.d.). Retrieved from Keil Web site: http://www.keil.com/dd/chip/6103.htm

Tushar. (2013, January 25). Controlling servo motor with AVR . Retrieved from Embedds: http://www.embedds.com/controlling-servo-motor-with-avr/

Tushar. (2013, July 26). Interfacing shift register with AVR . Retrieved from Embedds Seb site: http://www.embedds.com/interfacing-shift-register-with-avr/

Tushar. (2013, August 4). Interfacing Ultrasonic Rangefinder with AVR . Retrieved from Embedds: http://www.embedds.com/interfacing-ultrasonic-rangefinder-with-avr/

1 man-month equals to 22 man-day