|  | MINISTRY OF EDUCATION AND TRAINING |
| --- | --- |

FPT UNIVERSITY

Capstone Project Document

Designing and making A Lynxmotion A-POD robot controller

Hexapod Team. Group Members; - Phan Anh Dũng Cường - Nguyễn Minh Quân. Supervisor; M.Si. Trần Khánh Ninh. Ext Supervisor. Capstone Project code; APOD.

- Ho Chi Minh City, 09/2013 -

# Table of Contents

Table of Contents	2

List of Figures	4

List of Tables	6

A.	INTRODUCTION	9

I. PROJECT INFORMATION	9

II. TEAM MEMBER	9

III. APOD INTRODUCTION	9

1. HEXAPOD	9

2. LYNXMOTION APOD	11

IV. EXISTING SOLUTIONS	12

V. PROJECT INTRODUCTION	13

VI. APPROACHES	14

VII. OTHER	14

1. WHY WE NEED APOD ROBOT?	14

2. PRODUCT	14

B.	SOFTWARE PROJECT MANAGEMENT PLAN (SPMP)	15

I. PROBLEM DEFINITION	15

1. NAME OF THIS CAPSTONE PROJECT	15

2. PROBLEM ABSTRACT	15

3. PROBLEM OVERVIEW	15

II. PROJECT ORGANIZATION	17

1. SOFTWARE PROCESS MODEL	17

2. ROLES AND RESPONSIBILITIES	18

3. TOOL AND TECHNIQUES	18

III. PROJECT MANAGEMENT PLAN	19

1. Task	19

2. Task sheet	21

3. All meeting minutes	22

IV. CODING CONVENTION	22

C.	SOFTWARE REQUIREMENTS SPECIFICATION (SRS)	23

I. USER REQUIREMENT SPECIFICATIONS	23

II. SYSTEM REQUIREMENT SPECIFICATIONS	23

1. Hardware requirement	23

2. Software interface	23

3. External interface	23

4. Functional requirement	23

5. Non-functional requirement	24

III. USE-CASE DIAGRAM AND USE-CASE SPECIFICATIONS	24

1. Use-case Diagram	24

2. Use-case Specifications	25

IV. SOFTWARE SYSTEM ATTRIBUTES	51

1. Reliability	51

2. Availability	52

3. Security	52

4. Maintainability	52

5. Portability	52

6. Performance	52

V. ENTITY RELATIONSHIP DIAGRAM	53

D.	SOFTWARE DESIGN DESCRIPTION (SDD)	53

I. DESIGN OVERVIEW	53

1. Introduction	53

2. Architecture	54

3. User interface	54

II. COMPONENTS	55

1. Central processing:	55

2. Servos control & Sensor reader	57

III. IMPLEMENTATION APPROACHES	59

1. Algorithms and Mechanism	59

2. SSC32 Board	73

3. STM32F4 Discovery Board	75

IV. ACTIVITY AND CLASS STRUCTURE	75

1. Activity	75

2. Class Structure	79

V. SEQUENCE DIAGRAM	80

1. Embedded Controller	80

2. Use-cases Sequences	84

VI. USER INTERFACE DESIGN	97

1. Live Control	97

2. Sequence Player	98

3. Configuration	99

4. Object Extractor	100

E.	SOFTWARE TEST DOCUMENTATION (STD)	101

F.	SOFTWARE USER’S MANUAL (SUM)	101

G.	APPENDIX	101

I. Glossary	101

II. References	101

# List of Figures

Figure A-1 CH3-R Walking Robot	10

Figure A-2 AH2 Walking Robot	10

Figure A-3 Lynxmotion APOD	11

Figure A-4 Flowbotics Studio example 1	12

Figure A-5 Flowbotics Studio example 2	12

Figure A-6 Lynxmotion Visual Sequences	13

Figure B-1 System Boundary Overview	16

Figure B-2 Spriral model	17

Figure B-3 Project plan	22

Figure B-4 Project plan	22

Figure C-1: Use cases diagram	24

Figure C-2 Add State Use-case diagram	25

Figure C-3 Remove States Use-case diagram	26

Figure C-4 Import States Use-case diagram	27

Figure C-5 Export States Use-case diagram	29

Figure C-6 Normal input Use-case diagram	30

Figure C-7 Gamepad Input Use-case diagram	31

Figure C-8 Move Forward Use-case diagram	33

Figure C-9 Move Backward Use-case diagram	34

Figure C-10 Turn Left Use-case diagram	36

Figure C-11 Turn Right Use-case diagram	37

Figure C-12 Body Lift Use-case diagram	39

Figure C-13 Body Drop Use-case diagram	40

Figure C-14 Squeeze Left Use-case diagram	41

Figure C-15 Squeeze Right Use-case diagram	43

Figure C-16 Manual Grip Use-case diagram	44

Figure C-17 Auto Grip Use-case diagram	46

Figure C-18 Choose Target Use-case diagram	47

Figure C-19 Change Target Use-case diagram	49

Figure C-20 Lock Target Use-case diagram	50

Figure C-21	53

Figure D-1 APOD Controller System Architecture	54

Figure D-2 Live Control User interface design	54

Figure D-3 Sequence Player user interface design	55

Figure D-4 Bluetooth Communication module HC05	56

Figure D-5 STM32F4 Discovery Board	57

Figure D-6 HC 645MG Servo	58

Figure D-7 SSC32 Board	58

Figure D-8 HCSR04 Distance Sensor	59

Figure D-9 RC Servo block	60

Figure D-10 Servo Signal illustration	60

Figure D-11 HC 645MG Signal Details	61

Figure D-12 HC-SR04 Specifications	62

Figure D-13 HC-SR04 Best practice angle	62

Figure D-14 HC-SR04 Operating sequence	63

Figure D-15 Tripod Gait illustration	64

Figure D-16 Wave Gait illustration	65

Figure D-17 Wave Gait step order	65

Figure D-18 Ripple Gait illustration	66

Figure D-19 APOD leg position block	67

Figure D-20 APOD leg swing diagram	68

Figure D-21 APOD Leg full step diagram	68

Figure D-22 Tripod A at Front position	69

Figure D-23 Tripod A at Neutral position	70

Figure D-24 Tripod A at Rear position	70

Figure D-25 Tripod A at Left position	72

Figure D-26 Tripod A at Right position	72

Figure D-27 SSC32 Schematic	74

Figure D-28	76

Figure D-29	77

Figure D-30	78

Figure D-31	79

Figure D-32	80

Figure D-33	81

Figure D-34	82

Figure D-35	83

Figure D-36	84

Figure D-37	85

Figure D-38	85

Figure D-39	86

Figure D-40	87

Figure D-41	88

Figure D-42	89

Figure D-43	89

Figure D-44	90

Figure D-45	90

Figure D-46	91

Figure D-47	91

Figure D-48	92

Figure D-49	92

Figure D-50	93

Figure D-51	94

Figure D-52	95

Figure D-53	96

Figure D-54 Live Control Interface Design	97

Figure D-55 Sequence Player Interface Design	98

Figure D-56 Configuration Interface Design	99

Figure D-57 Object Extractor Interface Design	100

# List of Tables

Table A-1 Team Members	9

Table B-1  Roles and Responsibilities	18

Table B-2 Initiating and Planning	19

Table B-3 Software Requirement Analysis	19

Table B-4 Creating Software Design Description	20

Table B-5 Coding	20

Table B-6 Testing	20

Table B-7 Deployment	21

Table C-1	26

Table C-2	27

Table C-3	29

Table C-4	30

Table C-5	31

Table C-6	33

Table C-7	34

Table C-8	35

Table C-9	37

Table C-10	38

Table C-11	40

Table C-12	41

Table C-13	43

Table C-14	44

Table C-15	46

Table C-16	47

Table C-17	48

Table C-18	50

Table C-19	51

Table D-1	67

Table D-2 APOD Leg full forward step	69

Table D-3 Forward Sequence	71

Table D-4 Backward Sequence	71

Table D-5 Turn Left Sequence	73

Table D-6 Turn Right Sequence	73

Table D-7 Live Control Interface Design Description	97

Table D-8 Sequence Player Interface Design Description	98

Table D-9 Configuration Interface Design Description	99

Table D-10 Object Extractor Interface Design Description	100

# INTRODUCTION

## PROJECT INFORMATION

- Project name: Lynxmotion A-Pod  robot controller

- Project code: APOD

- Product type: Embedded robot controller

- Timeline: from September 2013 to December 2013

## TEAM MEMBER

| No. | Name | Role | Contact |
| --- | --- | --- | --- |
| 1 | Phan Anh Dũng Cường | Leader | - Phone: 0972708318 - Email: cuongpadse60774@fpt.edu.vn |
| 2 | Nguyễn Minh Quân | Member | - Phone: 0915699635 - Email: quannm60344@fpt.edu.vn |

Table A-1 Team Members

## APOD INTRODUCTION

### HEXAPOD

The trademarked name “hexapod” (by Geodetic Technology) was originally for Stewart platforms used in machine tools. However, the term is now used for 6-jack platforms outside of the machine tool area, since it simply means "six legs".

In the scope of this document, “hexapod” will represent a structure with six legs which can be used for variety of Robotics application.

Variety of robotics hexapod:

Figure A-1 CH3-R Walking Robot

Figure A-2 AH2 Walking Robot

### LYNXMOTION APOD

Figure A-3 Lynxmotion APOD

- Insect inspired body.

- Assembly with 25 servos (motors) and PVC materials to form 6 legs, mandible grippers (with force sensor feature) and a tail.

- The three DOF (degree of freedom) leg design help the APOD moving in a variety of directions.

- Servos are directly controlled by SSC-32 board which receive command via RS232 interface from Bot board II / PC or any other devices communicate using RS232.

## EXISTING SOLUTIONS

- Flowbotics Studio: $39.9

Figure A-4 Flowbotics Studio example 1

Figure A-5 Flowbotics Studio example 2

- Lynxmotion Visual Sequences: $39.94

Figure A-6 Lynxmotion Visual Sequences

## PROJECT INTRODUCTION

This project focus on programming on microprocessor unit (MCU) to developing, control the A-Pod robot. The BASIC is let him move forward, backward, right, left. Thus, we concern on control over Bluetooth.  Through Camera put on robot control it/ or automatic function, connect with Sensor to discover obstacles.

Abstracts purpose: Design the controller board motor servo using microcontroller to control Lynxmotion A-Pod  robot  with available mechanical. Implement the hardware and software on PC to control Lynxmotion. The system requires Microcontroller ARM LPC,AT91SAM,PIC,AVR. Finding critical points and find best way algorithms.

## APPROACHES

- Building SSC32 firmware with ATMega328p microcontroller to control 25 servos.

- Using Bot Board II  as universal sensor reader to serve automation purposes (additional sensors will be needed).

- Additional STM32F4 Discovery using ARM will be used as a central processing module which communicate with other device via Bluetooth.

- IP Camera to provide an insight view for user.

- A friendly HMI program will be provided for user to control the APOD with ease.

## OTHER

### WHY WE NEED APOD ROBOT?

The main purpose of A-Pod Robot is to be used at home with variable simple tasks. The Project ‘s scope is to control A-Pod from a distance via a HCI program on PC (or mobile devices). A-Pod can perform task without the present of human and report back working  information when needed.

### PRODUCT

- HMI Program on PC (Possible extend: Android/IOS).

- Servo Control Module Software (SSC32 firmware).

- Sensor Service Module  Software (Bot Board II firmware)

- Central Processing Module Software (STM32F4 Discovery).

# SOFTWARE PROJECT MANAGEMENT PLAN (SPMP)

## PROBLEM DEFINITION

### NAME OF THIS CAPSTONE PROJECT

  - Project name: Lynxmotion A-Pod  robot controller

  - Project code: APOD

  - Product type: Embedded robot controller

### PROBLEM ABSTRACT

As mentioned above, this project will focus on implement a new control system for APOD using Bluetooth/wireless. The main purpose of new controller system is to make the APOD more flexible. In order to achieve that purpose, the final product should have those characteristics:

  - Friendly HMI interface.

  - Easy to use, easy to learn.

  - Different user experience: comfort and high interaction.

  - Adaptable to new mechanical constraint, prepared for complete automation.

### PROBLEM OVERVIEW

#### The Current System

Current APOD control system can be control using either USB-to-serial cable or an PS2 interface. The SSC32 will be responsible for direct controlling of motor servos.

- Advantages:

  - Direct command.

  - No delay due to no-subsystem needed (USB-to-serial control).

  - Simple architecture

- Limitation:

  - Very limited range of control due to “wire” problem

  - A considerable amount of wire/cable can be obstacles for APOD movement.

  - Domain or mechanical knowledge is required to perform.

  - On-sight performance: APOD must stay insight of user viewpoint.

#### The Propose System

The system consists of 4 module:

  - Servos Control Module (SSC-32): Directly control servo ‘s movement.

  - Sensor Service Module (Bot Board II): receive all kind of sensor signal and value.

  - Central Processing Module (STM32F4 Discovery) plus Bluetooth module

  - HMI software on PC

Main features will be provided:

  - Basic movement: forward, backward, turn left, turn right.

  - High interactive user interface.

#### Boundaries of The System

Figure B-1 System Boundary Overview

  - The APOD can perform only within Bluetooth or Wireless device ‘s range

  - Any interfere while receiving command (transmitting via Bluetooth) will cause the APOD to misbehave or unpredictable movement.

#### Developing Environment

##### Hardware environment:

- Lyxnmotion APOD

- IP camera, Bluetooth device, Distance sensor.

- 4 laptops with appropriate configuration for embedded development.

##### Software environment:

- Developing environment ARV studio 4 for main board programming.

- Developing environment Visual studio 2012 with C# language for user interface programming.

## PROJECT ORGANIZATION

### SOFTWARE PROCESS MODEL

The process model used for developing this project is Spiral Model.

The spiral model is a software development process combining elements of both design and prototyping-in-stages, in an effort to combine advantages of top-down and bottom-up concepts. Also known as the spiral lifecycle model (or spiral development), it is a systems development method (SDM) used in information technology (IT). This model of development combines the features of the prototyping and the waterfall model. The spiral model is intended for large, expensive and complicated projects.

Figure B-2 Spriral model

### ROLES AND RESPONSIBILITIES

| No. | Full name | Team Role | Responsibilities |
| --- | --- | --- | --- |
| 1 | Trần Khánh Ninh | Supervisor | Define business Support in technical issues |
| 1 | Phan Anh Dũng Cường | Team Leader | Prioritize work Answer question and deliver direction Facilitate productivity – maximize team performance Complete all individual work |
| 3. | Nguyễn Minh Quân | Team Member | Commit individual product on time Support each other to complete team work |

Table B-1  Roles and Responsibilities

### TOOL AND TECHNIQUES

#### For Development

##### Hardware environment:

- Lynxmotion APOD

- IP camera, Bluetooth device, Distance sensor.

- 4 laptops with appropriate configuration for embedded development.

##### Software environment:

- Developing environment ARV studio 4 for main board programming.

- Developing environment Visual studio 2010 with C# language for user interface programming.

- Developing environment Keilc studio for program ARM.

#### For Management

- Microsoft Project: Task tracking

- Tortoise SVN: Source version control

- Google Code: Connect, synchronize source code and documents

- Beyond Compare: Manage changes in source files

#### For Communication

- Gmail report

- Skype chat system

- CMS system

## PROJECT MANAGEMENT PLAN

### Task

#### Initiating and Planning:

| Description | Registering project, kick-off meeting and planning |
| --- | --- |
| Output | Registered project, team spirit, overview plan |
| Deliverables | Draft project plan |
| Effort (man-month) | 0.8 mm |
| Dependencies and Constrains | N/A |
| Risks | Some members may be absent |

Table B-2 Initiating and Planning

#### Software Requirement Analysis

| Description | Analyzing software requirements based on available hexapod’s hardware to create software requirements specification document |
| --- | --- |
| Output | Software Requirement Specification document |
| Deliverables | SRS document file |
| Effort (man-month) | 1.5 mm |
| Dependencies and Constrains | N/A |
| Risks | - Lack of knowledge about hexapod’s hardware |

Table B-3 Software Requirement Analysis

#### Creating Software Design Description

| Description | Designing the controller for hexapod robot based on actual requirements |
| --- | --- |
| Output | Architecture design, circuits diagram, board diagram, algorithms and design specification |
| Deliverables | SDD document |
| Effort (man-month) | 3.0 mm |
| Dependencies and Constrains | Completion of SRS |
| Risks | - Choosing inappropriate algorithms and design patterns - The hardware is hard to maintain - Causing high coding efforts |

Table B-4 Creating Software Design Description

#### Coding

| Description | Designing the controller for hexapod robot based on actual requirements |
| --- | --- |
| Output | Architecture design, circuits diagram, board diagram, algorithms and design specification |
| Deliverables | SDD document |
| Effort (man-month) | 3.0 mm |
| Dependencies and Constrains | Completion of SRS |
| Risks | - Choosing inappropriate algorithms and design patterns - The hardware is hard to maintain - Causing high coding efforts |

Table B-5 Coding

#### Testing

| Description | Creating test case and execute test |
| --- | --- |
| Output | Test plan, test case document, test report, all tested modules and tested system |
| Deliverables | Test plan, test report |
| Effort (man-month) | 2.2 mm |
| Dependencies and Constrains | Completion of SRS, SDD, coding |
| Risks | - Lack of professional testers in team - Unit test may not be performed thoroughly causing spending many efforts in system test phase. - Hardware ‘s limitations and errors - ES testing is different with IS testing |

Table B-6 Testing

#### Deployment

| Description | Deploying system include : hexapod robot and hexapod controller simulator |
| --- | --- |
| Output | Software packages, user manual |
| Deliverables | Software packages, user manual |
| Effort (man-month) | 0.3 mm |
| Dependencies and Constrains | Completion of all other tasks |
| Risks | - Hardware’s errors |

Table B-7 Deployment

### Task sheet

Figure B-3 Project plan

Figure B-4 Project plan

### All meeting minutes

<to be updated>

## CODING CONVENTION

The following rules follow:

- The standard rules for developing application using C/C++ (http://users.ece.cmu.edu/~eno/coding/CCodingStandard.html)

# SOFTWARE REQUIREMENTS SPECIFICATION (SRS)

## USER REQUIREMENT SPECIFICATIONS

- A-Pod should be able to walk freely in any direction.

- Controlling via a Serial connection like Bluetooth, or directly through a PS2 controller.

- A-Pod should be able to grab small things like a Coke.

- A-Pod should be able to recording video.

- A-Pod should be able to detect objects stand (within 30cm) in the ways and response back.

- Controller should be simple and easy.

- After receive movement signal, A-Pod should move within 2 seconds

## SYSTEM REQUIREMENT SPECIFICATIONS

### Hardware requirement

- 25 HS-645MG servo

- 1 SSC-32 servo controller

- 1 Bot Board II with Basic Atom Pro 28 microcontroller

- 1 HC-SR04 supersonic sensor

- 1 STM32F4 ARM

- Laptops with Bluetooth Device

- 1 PS2 controller

- Few serial cables, adapters , jumpers…

### Software interface

- Menu-driven design with : button, dialog box, screen.

- Button : movement button on the right, setting like PS2 controller.

- Dialog box : on the left, display A-Pod ‘s currently speed, object stand in way and distance.

- Screen : center of menu controller, display view from camera.

### External interface

- PS2 Controller

### Functional requirement

- Direction control : user can direct the A-Pod to turn left, right, go forward or backward.

- Camera view control : user can see the view sending by the A-Pod camera.

- Grab control : user can direct the A-Pod to grab things.

- Remote control : enable remote controlling, help the user control the A-Pod through a programmed Bluetooth device.

### Non-functional requirement

- The A-Pod should response and move after receiving order within 2 seconds.

- The longest range for leg movement should be 30 centimeter, 35 degree.

- Detected range should be larger than 2 meter.

- View from camera should be refresh  every 0.5 second.

- The A-Pod must be compatible with the following batteries and chargers:

  - NiCad & Ni-MH Universal Smart Charger (USC-02)

  - Volt Ni-MH 2800mAh Battery Pack

- All cable, jumper, adapter should be corrected connect

## USE-CASE DIAGRAM AND USE-CASE SPECIFICATIONS

### Use-case Diagram

Figure C-1: Use cases diagram

### Use-case Specifications

#### Sequences Control

##### Add State

Figure C-2 Add State Use-case diagram

| ADD STATE USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC001 | Use-case Version |  | 1.0 |
| Use-case Name | Add State |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Add a state (move) to sequence Goal: New state is added to sequence Triggers User click [Add] button Preconditions: - Sequence Player feature is selected - Interval (if required) stay within valid range Post Conditions: NONE Main Success Scenario: - User select move to add to sequence - Enter interval value (if available) - Click [Add] Alternative Scenario: NONE Exceptions: NONE Relationships: NONE |  |  |  |  |

Table C-1

##### Remove states

Figure C-3 Remove States Use-case diagram

| REMOVE STATES USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC002 | Use-case Version |  | 1.0 |
| Use-case Name | Remove States |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Remove selected states from current sequence Goal: All selected state to be removed Triggers User click [Remove] Button Preconditions: - Sequence Player feature is selected Post Conditions: NONE Main Success Scenario: - User select all move to be removed from sequence - Click [Remove] Alternative Scenario: NONE Exceptions: NONE Relationships: NONE |  |  |  |  |

Table C-2

##### Import States

Figure C-4 Import States Use-case diagram

| IMPORT STATES USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC003 | Use-case Version |  | 1.0 |
| Use-case Name | Import States |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Import existing sequence from file Goal: Current sequence is replace by new one from file Triggers User click [Import] Button Preconditions: - Sequence Player feature is selected Post Conditions: NONE Main Success Scenario: - Click [Import] button - Browse to location contain sequence file (*.xml) - Select file & click [Ok] - Validate new sequence content - Replace current sequence if valid Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-3

##### Export States

Figure C-5 Export States Use-case diagram

| EXPORT STATES USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC004 | Use-case Version |  | 1.0 |
| Use-case Name | Export States |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Export current sequence to file Goal: New Sequence file Triggers User click [Export] button Preconditions: - Sequence Player feature is selected Post Conditions: NONE Main Success Scenario: - Click [Export] button - Browse to location contain sequence file (*.xml) - Click [Ok] Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-4

#### Direct Control

##### Mode Selection

###### Normal Input

Figure C-6 Normal input Use-case diagram

| NORMAL INPUT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC005 | Use-case Version |  | 1.0 |
| Use-case Name | Normal Input Selection |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Using normal input method (keyboard or Mouse) Goal: Normal input is selected Triggers User click [Normal Mode] button Preconditions: - Live control feature is selected Post Conditions: NONE Main Success Scenario: - User click [Normal Mode] button - Configuration is checked - Establish Bluetooth connection Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-5

###### Gamepad Input

Figure C-7 Gamepad Input Use-case diagram

| GAMEPAD INPUT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC006 | Use-case Version |  | 1.0 |
| Use-case Name | Gamepad Input Selection |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Using Gamepad input method (Gamepad) Goal: Gamepad input is selected Triggers User click [Gamepad Mode] button Preconditions: - Live control feature is selected - Gamepad is connected Post Conditions: NONE Main Success Scenario: - User click [Gamepad Mode] button - Configuration is checked - Gamepad device connection is checked - Establish Bluetooth connection Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-6

##### Movement Control

###### Move Forward

Figure C-8 Move Forward Use-case diagram

| MOVE FORWARD USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC007 | Use-case Version |  | 1.0 |
| Use-case Name | Move Forward |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to move forward Goal: APOD walking in forward direction Triggers User click [Forward] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Forward] button () Alternative Scenario: - Alt 1: press “W” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-7

###### Move Backward

Figure C-9 Move Backward Use-case diagram

| MOVE BACKWARD USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC008 | Use-case Version |  | 1.0 |
| Use-case Name | Move Backward |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to move Backward Goal: APOD walking in Backward direction Triggers User click [Backward] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Backward] button () Alternative Scenario: - Alt 1: press “S” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-8

###### Turn Left

Figure C-10 Turn Left Use-case diagram

| TURN LEFT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC009 | Use-case Version |  | 1.0 |
| Use-case Name | Turn Left |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to turn to the left Goal: APOD turn in Left direction Triggers User click [Turn Left] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Turn Left] button () Alternative Scenario: - Alt 1: press “A” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-9

###### Turn Right

Figure C-11 Turn Right Use-case diagram

| TURN RIGHT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC010 | Use-case Version |  | 1.0 |
| Use-case Name | Turn Right |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to turn to the Right Goal: APOD turn in Right direction Triggers User click [Turn Right] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Turn Right] button () Alternative Scenario: - Alt 1: press “D” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-10

###### Body Lift

Figure C-12 Body Lift Use-case diagram

| BODY LIFT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC011 | Use-case Version |  | 1.0 |
| Use-case Name | Body Lift |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to Lift up Goal: APOD Lift body up Triggers User click [R1] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected Post Conditions: NONE Main Success Scenario: - User click on [R1] button () Alternative Scenario: - Alt 1: press “U” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-11

###### Body Drop

Figure C-13 Body Drop Use-case diagram

| BODY DROP USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC012 | Use-case Version |  | 1.0 |
| Use-case Name | Body Drop |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to Drop down Goal: APOD Drop body Down Triggers User click [R2] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected Post Conditions: NONE Main Success Scenario: - User click on [R1] button () Alternative Scenario: - Alt 1: press “O” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-12

###### Squeeze Left

Figure C-14 Squeeze Left Use-case diagram

| SQUEEZE LEFT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC013 | Use-case Version |  | 1.0 |
| Use-case Name | Squeeze Left |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to squeeze to the left Goal: APOD Drop Squeeze left Triggers User click [Turn Left] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L2] button is pressed Post Conditions: NONE Main Success Scenario: - User press [L2] button () - User click on [Turn Left] button () Alternative Scenario: - Alt 1: press “A” key on keyboard while pressing “E” key (if Normal input is selected) - Alt 2: press () key on gamepad while pressing (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-13

###### Squeeze Right

Figure C-15 Squeeze Right Use-case diagram

| SQUEEZE RIGHT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC014 | Use-case Version |  | 1.0 |
| Use-case Name | Squeeze Right |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to squeeze to the right Goal: APOD Drop Squeeze right Triggers User click [Turn Right] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L2] button is pressed Post Conditions: NONE Main Success Scenario: - User press [L2] button () - User click on [Turn Left] button () Alternative Scenario: - Alt 1: press “D” key on keyboard while pressing “E” key (if Normal input is selected) - Alt 2: press () key on gamepad while pressing (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-14

###### Manual Grip

Figure C-16 Manual Grip Use-case diagram

| MANUAL GRIP USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC015 | Use-case Version |  | 1.0 |
| Use-case Name | Manual Grip |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Command APOD to Grip Mandibles Goal: APOD Mandibles Grip Triggers User click [Grip] button Preconditions: - Live control feature is selected - Bluetooth connection is established - Normal input mode or Gamepad input mode is selected - [L1] and [L2] is not pressed. Post Conditions: - APOD stop when button is released Main Success Scenario: - User click on [Grip] button () Alternative Scenario: - Alt 1: press “I” key on keyboard (if Normal input is selected) - Alt 2: press () key on gamepad (if Gamepad input is selected) Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-15

#### Object tracking

##### Auto Grip

Figure C-17 Auto Grip Use-case diagram

| AUTO GRIP USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC016 | Use-case Version |  | 1.0 |
| Use-case Name | Auto Grip |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Auto detect object in range and move forward to grab it. Goal: Object is grabbed Triggers User click [AUTO GRIP] button Preconditions: - Live control feature is selected - Bluetooth connection is established Post Conditions: NONE Main Success Scenario: - User click [AUTO GRIP] button - Auto grip command is sent to APOD Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-16

##### Choose Target

Figure C-18 Choose Target Use-case diagram

| CHOOSE TARGET USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC017 | Use-case Version |  | 1.0 |
| Use-case Name | Choose Target |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Enter tracking mode, choosing target to track Goal: New tracking template is selected Triggers User click [Object Tracking] button Preconditions: - Configuration is initiated - Bluetooth connection is established - IP camera stream is working Post Conditions: NONE Main Success Scenario: - [Object Tracking] button is clicked - Object extractor is showed - User select tracking template from capture frame. - User click [OK] Alternative Scenario: NONE Exceptions: - IO Exception - Time Out Exception Relationships: NONE |  |  |  |  |

Table C-17

##### Change Target

Figure C-19 Change Target Use-case diagram

| ADD POINT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC018 | Use-case Version |  | 1.0 |
| Use-case Name | Change Target |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Change current tracking template Goal: New tracking template is selected Triggers User click [Change target] button Preconditions: - Configuration is initiated - Bluetooth connection is established - IP camera stream is working - Tracking mode is selected Post Conditions: NONE Main Success Scenario: - [Change Target] button is clicked - Object extractor is showed - User select tracking template from capture frame. - User click [OK] Alternative Scenario: NONE Exceptions: - IO Exception - Time Out Exception Relationships: NONE |  |  |  |  |

Table C-18

##### Lock Target

Figure C-20 Lock Target Use-case diagram

| ADD POINT USE CASE SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Use-case No. | UC019 | Use-case Version |  | 1.0 |
| Use-case Name | Lock Target |  |  |  |
| Author | CuongPAD |  |  |  |
| Date | 05/10/2013 | Priority | Normal |  |
| Actor: User Summary: Start tracking target Goal: Reach target (within 60 cm) Triggers User click [Lock] button Preconditions: - Configuration is initiated - Bluetooth connection is established - IP camera stream is working - Tracking template was set Post Conditions: NONE Main Success Scenario: - User click on [Lock] button - APOD keep moving forward (with adjustment) until target is reached - User decide to Grab object or not. Alternative Scenario: NONE Exceptions: - IO Exception Relationships: NONE |  |  |  |  |

Table C-19

## SOFTWARE SYSTEM ATTRIBUTES

### Reliability

- Easy to upgrade firmware.

- The hexapod controller can be replaced easily by loaded into chip if the controller has problem.

- The APOD system is guarantee by quality testing in:

  - Stability constraints.

  - Functionality.

  - Reliability.

- It’s mostly depending on hardware reliability.

- Small error margin when moving.

### Availability

- In case of electrical incident, the hexapod system will be shut down and reset automatically.

- Controller has the flexibility that allows changes in hardware design.

- Hexapod Controller can actuate the 6-legs forces to properly position the mobile plate given a desired trajectory.

### Security

- N/A

### Maintainability

- N/A

### Portability

- The hexapod controller is depend on hardware so that hexapod system do not have the portability attribute. However, the hexapod controller can easily be loaded into chips and use in different motors.

- The development environment and the language constructs used ensure portability as much as possible to avoid the limitation of software’s changes each time the hardware is upgraded or replaced.

### Performance

- Hexapod controller can control multi-motors in the same time.

- The current hardware can move correctly

## ENTITY RELATIONSHIP DIAGRAM

Figure C-21

# SOFTWARE DESIGN DESCRIPTION (SDD)

## DESIGN OVERVIEW

### Introduction

This section is created to introduce and give a brief overview of the system design.

The followings information are given in this section:

- System Architecture Design : the system overall structure

- Components Description : detailed info of hardware components

- Class Diagram : entity & attributes

- Sequence Diagram : describe the flow of events

- User Interface Design

### Architecture

Figure D-1 APOD Controller System Architecture

### User interface

Figure D-2 Live Control User interface design

Figure D-3 Sequence Player user interface design

## COMPONENTS

### Central processing:

#### Bluetooth module (HC 05)

Communication module (using Bluetooth) to transmit and receive commands from Controller devices (PC)

Figure D-4 Bluetooth Communication module HC05

#### STM32F4 Discovery

The Center Processing module to control all APOD movements or reaction to a specific event (command or obstacle).

Figure D-5 STM32F4 Discovery Board

### Servos control & Sensor reader

#### HG-645MG Servo

- Metal gear servo that provided the base of APOD movement.

Figure D-6 HC 645MG Servo

#### SSC32 with ATMEGA328P

- A servo controller : contains 32 pin channels of 1uS resolution servo control

- For further information, followings this link : Lynxmotion SSC32

Figure D-7 SSC32 Board

#### Ultrasonic ranging module (HC SR04)

- Ultrasonic module using for distance measurement, discovering obstacles.

Figure D-8 HCSR04 Distance Sensor

## IMPLEMENTATION APPROACHES

### Algorithms and Mechanism

#### RC Servo Control

##### HG-645MG Specification

| Motor Type: | 3 Pole |
| --- | --- |
| Bearing Type: | Dual Ball Bearing |
| Speed (4.8V/6.0V): | 0.24 / 0.20 |
| Torque oz./in. (4.8V/6.0V): | 107 / 133 |
| Torque kg./cm. (4.8V/6.0V): | 8.0 / 10.0 |
| Size in Inches: | 1.59 x 0.77 x 1.48 |
| Size in Millimeters: | 40.39 x 19.56 x 37.59 |
| Weight ounces: | 1.94 |
| Weight grams: | 55 |

##### How does it work?

Unlike the AC/DC motor, Servo motor do not rotate continuously.  Servo motor rotation range usually fall within 90o to 180o (some special servo can rotate up to 360o) and keep holding the torque at the position as long as its power supply remain or until it receive signal to move the torque.

Most of Servos have 3 wires connector. One wire for power supply (4 – 6v). The second wire is for voltage ground. And the third wire is for receiving control signal.

Figure D-9 RC Servo block

The whole idea to control servo is sending the appropriate signal to servo so that it will move to the intended position. The signal to control servo, basically is a PWM (Pulse Width Modulation) signal of 50 Hz (or refresh rate: 20ms). It is the width of carrier pulse that decide the position of the torque (not the number of pulse).

Figure D-10 Servo Signal illustration

In order for the servo to keep the position, the signal must be maintain.

In short words, the pulse width of carrier signal will act as a coordinate value of the servo’s torque. Once the coordinate is changed, the servo constantly move the toque to the responding position discarding of its current position. The rotating speed will be depending on the manufacturer of the servo.

For the HG-645MG servo, the corresponding signal is showed below:

Figure D-11 HC 645MG Signal Details

##### Servo’s Speed Control

As we know, the servo will always rotate at a constant speed, for example the  servo HG-645MG speed is 60o/ 0.2s with 6V supply. With such speed applied to robot’s movement, it can affect the stability of the structure, especially for those in which mechanical part is not so much reliable. How do we slow it down?

The idea is not making the servo rotate a whole distance at once but make it rotate one step at a time until it reach its destination, in short it is “divide and conquer” method. For example, you want your HG-645MG servo to rotate 90o in 1 second and the current position of your servo is 0o. If you just apply the PWM with 1500 µs pulse width (1500 µs stand for 90o position), the servo will move to its destination in only 0.3 second. With the method mentioned above, we divide the whole 90o into ten 9o pieces (or 150 µs step as we call it). The signal pulse width will be modified 10 times, after each time the width will be added 150 µs more and waiting 100 ms until the next change happen. With this implementation, the servo will move 10x9o = 90o in 10x100 =  1000 ms = 1 s. The larger the divisor is, the smoother the servo will move.

For this project, the step is always fix at 20 µs and the waiting time fall in range of 10 ms to 20 ms depend on user purposes.

Let D be the travelling distance (in µs), T be the expected time for the move, δ be the waiting time for each step. We have the speed formula:

δ= T ×20D

#### Sensor Reader (HC-SR04)

##### HC-SR04 Specifications

Figure D-12 HC-SR04 Specifications

Figure D-13 HC-SR04 Best practice angle

##### How to read distance value?

The HC-SR04 has 4 pins:  VCC, GND, Trigger, Echo.

- VCC is used for power supply

- GND is used for voltage ground

- Trigger and Echo will perform as follow

Figure D-14 HC-SR04 Operating sequence

First the Trigger pin is pull high for more than 10 µs (10 µs pulse), then waiting for the Echo output to go high and start timing the length of the Echo pin staying high. At this point, a short ultrasonic pulse is transmitted, waiting to be reflected by an obstacle. When the reflected ultrasonic pulse is receive by senor, the Echo output will go low.

The time that Echo output staying high will be used to calculate the distance by following formula given by the manufacturer:

distance cm=pulse width (μs)58

Or

distance inch=pulse width (μs)148

#### Multi-legs Control

The description below is described in the following article by Oricom Technologies – “Analysis of Multi-Legged Animal + Robot Gaits” (2003).

##### Tripod Gait

The tripod gait is the best-known hexapod gait. Hexapod six legs is divide into 2 halves for to control. Each half (or tripod) consists of 3 legs: the front and rear leg of one side and the center leg of the opposite side. More specific:

- Tripod A: left front, right center and left rear

- Tripod B: right front, left center and right rear.

Figure D-15 Tripod Gait illustration

During walking cycle, the weight of the entire hexapod is shifted from one tripod to another. At a moment, there is always one tripod on the ground while another doing its cycle. “Since 3 legs are on the ground at all times, this gait is both "statically" and "dynamically" stable.” - (Oricom Technologies , 2003)

##### Wave Gait

The wave gait implementation make each leg in one side do it cycle after one another leg has finished its cycles and repeat it on the other side’s legs.

Figure D-16 Wave Gait illustration

For instance, the turn of leg taking step is as follow: Left Front -> Left Center -> Left Rear -> Right Front -> Right Center -> Right Rear

Figure D-17 Wave Gait step order

Since there is always 5 legs on the ground, this method is the most stable gait but in return, it is also the slowest gait due to its “one step at a time”.

##### Ripple Gait

“The final stride is the Ripple Gait. At first glance the timing of this gait looks somewhat complicated, however, the key to understanding is to recognize that, on each side a local wave comprising non-overlapping lift phases is being performed, and that the 2 opposite side waves are exactly 180 degrees out of phase with one another. For instance, if L3 and R3 are considered to represent the start of each local wave, then notice that R3 starts to move exactly in the middle of the L3-L2-L1 side wave” .” (Oricom Technologies , 2003)

Figure D-18 Ripple Gait illustration

##### Implementation Approaches

For this project, we will apply the tripod gait method to control the APOD walking movement. As we know, 6 legs of APOD will be divided into 2 tripods

We defined 2 tripod as follow:

- Tripod A: Left front, Right Center, Left Rear

- Tripod B: Right front, Left Center, Right Rear

###### Walking In Straight Line

Leg’s Position:

There are 6 basic positions for the APOD’s leg divive into 2 orientation.

- Vertical: High, Middle , Low

- Horizontal: Front, Center, Rear

Figure D-19 APOD leg position block

Each step (swing time) of the APOD leg is represented by a sequence of 4 consecutive states defined by the position of the leg in both team of vertical and horizontal positions.

For example, this is the forward step sequence:

| State | Vertical | Horizontal |
| --- | --- | --- |
| 0 | Low to Mid | Rear |
| 1 | Mid to High | Rear to Center |
| 2 | High to Mid | Center to Front |
| 3 | Mid to Low | Front |

Table D-1

Figure D-20 APOD leg swing diagram

Depend on the step direction (forward or backward) the sequence may differ: “Front” and “Rear” may be replaing each other’s position.

After making the step, the leg will return to its original position (compare to the body) while keeping the feet on the ground (stand time). This is the point the force was born to push the body moving in the opposite direction of the moving legs.

From the APOD view point, the whole operations of moving leg can be seen as the figure below:

Figure D-21 APOD Leg full step diagram

The whole operation of APOD’s leg can be seen as this sequence of states (Forward step) looping times to times:

| State | Vertical | Horizontal |
| --- | --- | --- |
| 0 | Low | Front to Center |
| 1 | Low | Center to Rear |
| 2 | Low | Rear |
| 3 | Low to Mid | Rear |
| 4 | Mid to High | Rear to Center |
| 5 | High to Mid | Center to Front |
| 6 | Mid to Low | Front |
| 7 | Low | Front |

Table D-2 APOD Leg full forward step

Tripod Position

According to the nature of  The Tripod Gait, every 3 legs in one tripod will always move and stop at the same time. In other words, their cycles are exactly the same phase with one another. The term tripod position is defined by the combination of its legs positions.

Let’s take a view of Tripod A (marked with red color) from the up top viewpoint for example:

- Position: Front (close to head)

Figure D-22 Tripod A at Front position

- Position: Center or Neutral

Figure D-23 Tripod A at Neutral position

- Position: Rear (far from head)

Figure D-24 Tripod A at Rear position

Since all Tripod A’s Legs making the forward step at once (moving from front to rear), there will be 2 pushing force from both side of the body, from the back to the head, that making the APOD moving forward. This is the fundamental of making the APOD moving forward. The backward move can be achieve by making the reverse step of forward step (the backward step).

Walking

Now before we can figure out the walking method of APOD, let’s examine the human walking method first. As we can see, the position of one leg in walking (or running) is always the opposite position of the other one. Or in short, their cycles is exactly 180o out of phase with one another. The similar mechanism is applied for The Tripod Gait walking method. Let Tripod A and B stand for 2 legs of human, then their cycles will always be 180o out of phase with the other one. In conclusion, the full sequence for 2 tripods of The Tripod Gait walking method is described as below

Forward Sequence

| State | Tripod A |  | Tripod B |  |
| --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal |
| 0 | Low | Front to Center | Mid to High | Rear to Center |
| 1 | Low | Center to Rear | High to Mid | Center to Front |
| 2 | Low | Rear | Mid to Low | Front |
| 3 | Low to Mid | Rear | Low | Front |
| 4 | Mid to High | Rear to Center | Low | Front to Center |
| 5 | High to Mid | Center to Front | Low | Center to Rear |
| 6 | Mid to Low | Front | Low | Rear |
| 7 | Low | Front | Low to Mid | Rear |

Table D-3 Forward Sequence

Backward Sequence

| State | Tripod A |  | Tripod B |  |
| --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal |
| 0 | Low | Rear to Center | Mid to High | Front to Center |
| 1 | Low | Center to Front | High to Mid | Center to Rear |
| 2 | Low | Front | Mid to Low | Rear |
| 3 | Low to Mid | Front | Low | Rear |
| 4 | Mid to High | Front to Center | Low | Rear to Center |
| 5 | High to Mid | Center to Rear | Low | Center to Front |
| 6 | Mid to Low | Rear | Low | Front |
| 7 | Low | Rear | Low to Mid | Front |

Table D-4 Backward Sequence

###### Turning Left – Right

As mention above, we already know 3 basic position of tripod: Front, Center, Rear. In this section, there are 2 more position: Left and Right.

The definition  of 2 new position can be described by tripod A as follow:

- Position: Left

Figure D-25 Tripod A at Left position

- Position: Right

Figure D-26 Tripod A at Right position

Let’s take a look when Tripod A making set :

- Left Front and Left Rear take the forward step

- Right Center take the backward step

There will be forces with 2 opposite direction on the APOD body. The left side force push the body forward and the right side force push the body backward. As a result of this combination. The APOD will turn from left to right. In other words, when the tripod making transition from left position to right position (through center position), the APOD will turn from left to right. And when the tripod making transition from right position to left position (through center position), the APOD will turn from right to left.

The combination sequence for turning the APOD:

Turn Left:

| State | Tripod A |  | Tripod B |  |
| --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal |
| 0 | Low | Left to Center | Mid to High | Right to Center |
| 1 | Low | Center to Right | High to Mid | Center to Left |
| 2 | Low | Right | Mid to Low | Left |
| 3 | Low to Mid | Right | Low | Left |
| 4 | Mid to High | Right to Center | Low | Left to Center |
| 5 | High to Mid | Center to Left | Low | Center to Right |
| 6 | Mid to Low | Left | Low | Right |
| 7 | Low | Left | Low to Mid | Right |

Table D-5 Turn Left Sequence

Turn Right:

| State | Tripod A |  | Tripod B |  |
| --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal |
| 0 | Low | Right to Center | Mid to High | Left to Center |
| 1 | Low | Center to Left | High to Mid | Center to Right |
| 2 | Low | Left | Mid to Low | Right |
| 3 | Low to Mid | Left | Low | Right |
| 4 | Mid to High | Left to Center | Low | Right to Center |
| 5 | High to Mid | Center to Right | Low | Center to Left |
| 6 | Mid to Low | Right | Low | Left |
| 7 | Low | Right | Low to Mid | Left |

Table D-6 Turn Right Sequence

### SSC32 Board

#### Schematic

Figure D-27 SSC32 Schematic

#### Technical Point

##### Atmega328 MCU

##### GPIO

##### USART

##### SPI

##### Shifter (74HC595)

##### SPI – Shifter interface

### STM32F4 Discovery Board

## ACTIVITY AND CLASS STRUCTURE

### Activity

#### APOD Controller (HMI Application)

Figure D-28

#### STM32F4 Discovery (Central processing board)

Figure D-29

#### SSC32 Board (Servos control and Sensor reader)

Figure D-30

### Class Structure

Figure D-31

## SEQUENCE DIAGRAM

### Embedded Controller

#### Forward

Figure D-32

#### Backward

Figure D-33

#### Turn Left

Figure D-34

#### Turn Right

Figure D-35

### Use-cases Sequences

#### Sequences Control

##### Add State

Figure D-36

##### Remove States

Figure D-37

##### Import States

Figure D-38

##### Export States

Figure D-39

#### Direct Control

##### Mode Selection

###### Normal Input

Figure D-40

###### Gamepad Input

Figure D-41

##### Movement Control

###### Forward

Figure D-42

###### Backward

Figure D-43

###### Turn Left

Figure D-44

###### Turn Right

Figure D-45

###### Body Lift

Figure D-46

###### Body Drop

Figure D-47

###### Squeeze Left

Figure D-48

###### Squeeze Right

Figure D-49

#### Object Tracking

##### Auto Grip

Figure D-50

##### Choose target

Figure D-51

##### Change Target

Figure D-52

##### Lock Target

Figure D-53

## USER INTERFACE DESIGN

### Live Control

4

2

3

10

13

12

14

11

8

9

7

6

5

1

0

Figure D-54 Live Control Interface Design

| Item NO. | Component Name | Description |
| --- | --- | --- |
| 0 | Menu | Select Features |
| 1 | VideoSourcePlayer | Display Camera IP ‘s View |
| 2 | Checkbox (Button) | Select Normal input mode |
| 3 | Checkbox (Button) | Select Gamepad input mode |
| 4 | Checkbox (Button) | Select object tracking mode |
| 5 | Button | Start streaming video from Camera |
| 6 | Image | Display tracking template |
| 7 | Button | Change Tracking template |
| 8 | Checkbox (Button) | Lock target – Start tracking |
| 9 | Button | Switch speed |
| 10 | Button | Auto grip |
| 11 | Keypad | Action selection |
| 12 | Keypad | Action bank selection |
| 13 | Keypad | Set mode/function |
| 14 | Keypad | Navigation Selection |

Table D-7 Live Control Interface Design Description

### Sequence Player

4

7

6

5

3

1

2

0

Figure D-55 Sequence Player Interface Design

| Item NO. | Component Name | Description |
| --- | --- | --- |
| 0 | Combo Box | Select action to add |
| 1 | Textbox | Movement interval |
| 2 | Button | Add action to sequence |
| 3 | Data Grid | Display current sequence content |
| 4 | Button | Remove selected states from sequence |
| 5 | Button | Export current sequence to file |
| 6 | Button | Import sequence from file |
| 7 | Button | Start playing the sequence |

Table D-8 Sequence Player Interface Design Description

### Configuration

85

75

65

5

44

35

25

1

0

Figure D-56 Configuration Interface Design

| Item NO. | Component Name | Description |
| --- | --- | --- |
| 0 | Textbox | Enter IP of camera |
| 1 | Textbox | Camera port to get stream |
| 2 | Textbox | Username of camera IP |
| 3 | Password Box | Password of camera IP |
| 4 | Combo Box | All available COM ports |
| 5 | Combo Box | Bluetooth baudrate selection |
| 6 | Button | Refresh list of Ports |
| 7 | Button | Accept information |
| 8 | Button | Cancel action |

Table D-9 Configuration Interface Design Description

### Object Extractor

25

1

0

Figure D-57 Object Extractor Interface Design

| Item NO. | Component Name | Description |
| --- | --- | --- |
| 0 | Image | Capture frame for template selection |
| 1 | Button | Accept new template |
| 2 | Button | Cancel action |

Table D-10 Object Extractor Interface Design Description

# SOFTWARE TEST DOCUMENTATION (STD)

# SOFTWARE USER’S MANUAL (SUM)

# APPENDIX

## Glossary

## References

All you need to know about AVR fuses . (2013, May 30). Retrieved from Embedds: http://www.embedds.com/all-you-need-to-know-about-avr-fuses/

Atmel. (n.d.). ATmega328. Retrieved from Atmel Web site: http://www.atmel.com/devices/atmega328.aspx?tab=documents

Electrical and Computer Engineering. (n.d.). C Coding Standard . Retrieved from Electrical and Computer Engineering Web site: http://users.ece.cmu.edu/~eno/coding/CCodingStandard.html

Frye, J. (2010, June 16). SSC-32 Manual. Retrieved from Lynxmotion Web site: http://www.lynxmotion.com/images/html/build136.htm

ITead Studio. (n.d.). Ultrasonic Ranging Module HC-SR04. Retrieved from ITead Studio Web site: http://imall.iteadstudio.com/im120628012.html

NXP . (n.d.). 74HC595PW. Retrieved from NXP Web site: http://www.nxp.com/products/logic/shift_registers/74HC595PW.html

Oricom Technologies . (2003, January). Analysis of Multi-Legged Animal + Robot Gaits. Retrieved from Oricom Technologies Web site: http://www.oricomtech.com/projects/leg-time.htm#Hex2

Tushar. (2013, January 25). Controlling servo motor with AVR . Retrieved from Embedds: http://www.embedds.com/controlling-servo-motor-with-avr/

Tushar. (2013, August 4). Interfacing Ultrasonic Rangefinder with AVR . Retrieved from Embedds: http://www.embedds.com/interfacing-ultrasonic-rangefinder-with-avr/

1 man-month equals to 22 man-day